
RAYSHARP HTTP API

Version 1.0.0
Preface

This document details the API of Raysharp video products. Programmers can access and configure Raysharp video products follows the API.

Catalog

2

1. Preface

2

2. Catalog

5

3. HTTP API Transaction

5

3.1Transaction

6

3.2Authentication

7

4. Camera

7

4.1 Stream

7

4.1.1 GetStream

7

4.2 CameraCap

7

4.2.1 GetCameraConfig

8

4.2.2 SetCameraConfig

10

4.3 VideoColor

10

4.3.1 GetVideoColorConfig

11

4.3.2 SetVideoColorConfig

11

4.3.3 Resolution

12

5. Live

12

5.1 Live General

12

5.1.1 GetLiveGeneralConfig

12

5.1.2 SetLiveGeneralConfig

13

5.2 OSD

13

5.2.1 GetOSDConfig

13

5.2.2 SetOSDConfig

14

5.3 VideoCover

14

5.3.1 GetVideoCoverConfig

14

5.3.2 SetVideoCoverConfig

15

6. Stream

15

6.1 MainStream

15

6.1.1 GetMainStreamConfig

16

6.1.2 SetMainStreamConfig

18

6.2 Sub Stream

18

6.2.1 GetSubStreamConfig

18

6.2.2 SetSubStreamConfig

21

6.3 Mobile Stream

21

6.3.1 GetMobileStreamConfig

21

6.3.2 SetMobileStreamConfig

23

6.4 Audio

23

6.4.1 GetAudioConfig

24

6.4.2 SetAudioConfig

24

7. NetWork

24

7.1 BasicConfig

24

7.1.1 GetBasicConfig

25

7.1.2 SetBasicConfig

26

7.2 3G

26

7.2.1 Get3GConfig

26

7.2.2 Set3GConfig

27

7.3 Wifi

27

7.3.1 GetWifiConfig

27

7.3.2 SetWifiConfig

28

7.4 DDNS

28

7.4.1 GetDDNSConfig

29

7.4.2 SetDDNSConfig

29

7.5 Email

29

7.5.1 GetEmailConfig

30

7.5.2 SetEmailConfig

31

7.6 P2P

31

7.6.1 GetP2PConfig

31

7.6.2 SetP2PConfig

31

7.7 RTSP

31

7.7.1 GetRTSPConfig

32

7.7.2 SetRTSPConfig

32

7.8 FTP

32

7.8.1 GetFTPConfig

33

7.8.2 SetFTPConfig

34

7.9 Platform

34

7.9.1 GetPlatformConfig

34

7.9.2 SetPlatformConfig

35

7.10 Mobile

35

7.10.1 GetMobileConfig

35

7.10.2 SetMobileConfig

35

7.11 GB28181

35

7.11.1 GetGB28181Config

36

7.11.2 SetGB28181Config

37

8. Alarm

37

8.1 IO Alarm

37

8.1.1 GetIOAlarmConfig

37

8.1.2 SetIOAlarmConfig

38

8.2 MotionDetect

38

8.2.1 GetMotionDetectConfig

38

8.2.2 SetMotionDetectConfig

40

8.3 PerimeterLine

40

8.3.1 GetPerimeterLineConfig

40

8.3.2 SetPerimeterLineConfig

41

8.4 OSCAlarm

41

8.4.1 GetOSCAlarm

42

8.4.2 SetOSCAlarm

43

8.5 ODAlarm

43

8.5.1 GetODAlarm

43

8.5.2 SetODAlarm

44

9. Record & Snap

44

9.1 General Record

44

9.1.1 GetGeneralRecConfig

45

9.1.2 SetGeneralRecConfig

45

9.2 Smart Record

45

9.2.1 GetSmartRecConfig

45

9.2.2 SetSmartRecConfig

46

9.3 Schedule

46

9.3.1 GetScheduleConfig

46

9.3.2 SetScheduleConfig

47

9.4 Snap

47

9.4.1 GetSnapConfig

47

9.4.2 SetSnapConfig

48

10. System

48

10.1 General

48

10.1.1 GetSysGeneralConfig

48

10.1.2 SetSysGeneralConfig

49

10.2 User

49

10.2.1 GetUserConfig

49

10.2.2 SetUserConfig

50

10.3 NTP

50

10.3.1 GetNTPConfig

50

10.3.2 SetNTPConfig

52

10.4 DST

52

10.4.1 GetDSTConfig

52

10.4.2 SetDSTConfig

53

11. Storage

53

11.1 CloudStorage

53

11.1.1 GetCloudStorageConfig

54

11.1.2 SetCloudStorageConfig

54

11.2 Storage Device

54

11.2.1 GetDevStorageConfig

55

11.2.2 SetDevStorageConfig

55

12. Advance

55

12.1 IP Filter

55

12.1.1 GetIPFilterConfig

55

12.1.2 SetIPFilterConfig

56

12.2 FtpUpgrade

56

12.2.1 GetFtpUpgradeConfig

56

12.2.2 SetFtpUpgradeConfig

57

13. Events

57

13.1 EventHandler

57

13.1.1 GetEventHandler



HTTP API Transaction

3.1Transaction
The HTTP API Transaction starts from a request from a client Application, usually a web browser. The request is processed by the web server on the Raysharp video products, then send the response back to the client application. The HTTP request is taken in GET form. If the request is successful, the Raysharp video product will return a HTTP header contains 200 OK. The HTTP Body will contain actual data or error message if an error occurs.
For describe convenience, we use some short words to instead the long expressions. The follows are several regulations:
The italics and bold will be replaced by the value behind the symbol “=”.
The URL must follow the standard way of writing a URL.(RFC_3986:Uniform Resource Identifiers (URI) Generic Syntax);that is ,spaces and other reserved characters (“;”, “/”, “?”, “:”, “@”, “=”, “+”, “,” and “$”) within a <paramName> or a <paramValue> must be replaced with %<ASCII hex>.For example ,the blank must be instead with %20.
To describe the range of the configuration, we use some symbols such as “[]”, “{}” and so on. For example :”[0-100]” denotes a integer not less than 0 and not larger than 100. “{0,1,2,3}” denotes the valid value of a integer among 0,1,2 and 3.
In the request and response, we use “[]” to denote an array. The index is usually a integer and start form 0.
The parameter value has several types: string, integer, bool and float.Integer is 32 bits.The range of bool is “true” and “false”.
The below is an example of a transaction:
	Request
	GET http://<ip>/action/getConfig?name=VideoColor

	Description
	Get VideoColor configuration.

	Response
	HTTP/1.1 200 OK
Content-Type:text/plain
Brightness=50

Contrast=50

Hue=50

Saturation=50



	Comment
	    ......
We can also request the single config.

For example:
Request : 
GET http://10.7.2.4/action/getConfig?name=VideoColor.Brightness

Response:
HTTP/1.1 200 OK
Content-Type:text/plain
VideoColor.Brightness=50


3.2Authentication
The Raysharp video product supplies two authentication ways: basic authentication and digest authentication. If the http request does not have  “Authorization” , the Raysharp video product returns 401, utill the http request has a legal authentication . 
For example:
1. When basic authentication, the Raysharp video product response:
401 Unauthorized

    WWW-Authenticate: Basic realm=”XXXXXX”

       Then the client encode the username and password with base64, send the following request:
Authorization: Basic VXZVXZ.
2. When digest authentication, the Raysharp video product response:
WWW-Authenticate: Digest realm="raysharp.cn", nonce="7a86c864b7fadff5457f90ec803a9182", qop="auth";

The client calculates the digest using username, password, nonce, realm and URI with MD5, then send the following request:
Authorization: Digest username="admin", realm="raysharp.cn", nc=00000001,cnonce="c36a271f",qop="auth"

nonce="7a86c864b7fadff5457f90ec803a9182",uri="action/getConfig?name=VideoColor", response="909a78946ae1cf2395f9a1fd8fbeb38d"

Camera

Camera API allows application to configure and view Raysharp video product settings.

4.1 Stream

4.1.1 GetStream
	URL Syntax
	rtsp://<ip>:<port>/<channelNo>/<typeNo>

	Comment
	<ip> :the IP address of the Raysharp video product.
<port >:the default port is 554. It can be omitted. 
<channelNo> :the channel number. It starts from ch00.
<typeNo> :the stream type. The <typeNo> of main stream is 0, sub stream  is 1.
For example, we request the extra stream 1 of channel 1, the URL is:
rtsp://192.168.6.67:554/ch01/0.


4.2 CameraCap
4.2.1 GetCameraConfig
	URL Syntax
	http://<ip>/action/getConfig?name=Camera

	Description
	camera options contain Backlight, ExposureSpeed, DayNightColor, and so on.

	Response
	Camera.IRCutMode=0
Camera.IRCutDelay=3
Camera.IRCutSensitive=1

Camera.IRFuncType=0

Camera.Mirror=0
Camera.Flip=0
Camera.Rotate=0
Camera.BackLightMode=1
Camera.BackLightLevel=1
Camera.R3dnrMode=1
Camera.R3dnrThreshTarget=50
Camera.DISEnable=1
Camera.DefogMode=1
Camera.DefogStrength=128
Camera.DwdrMode=1
Camera.DwdrStrength=50
Camera.GainControlMode=2
Camera.WBMode=0
Camera.Rgain=50
Camera.Ggain=50
Camera.Bgain=50
Camera.ShutterMode=0
Camera.ShutterSpeed=25
Camera.FlickerCtrl=0

	Comment
	ChannelNo = video channel index. Current default channel 0.


4.2.2 SetCameraConfig
	URL Syntax
	http://<ip>/action/setConfig?name=Camera&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	In below table, head =Camera[ChannelNo]

ChannelNo = video channel index.

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	Camera.IRCutMode
	char
	Range is {0, 1, 2, 3}
0 – auto.
1 – day.
2- night.
3 – video auto

	Camera.IRCutDelay
	char
	Range is [0 ~ 36]. unit : second

	Camera.IRCutSensitive
	char
	Range is {0, 1, 2}
0: High.
1: Middle.
2: Low.

	Camera.IRFuncType
	char
	Infrared function switch. 0: close, 1: auto.

	Camera.Mirror
	char
	Range is {0, 1}. Flip Horizontal  
0: close.

1: open.

	Camera.Flip
	char
	Range is {0, 1}. Flip vertical
0: close.

1: open.

	Camera.Rotate
	char
	Range is {0, 1, 2, 3}
0: does not rotate.
1: 90° clockwise.
2: 180° clockwise.

3: 270° clockwise.

	Camera.BackLightMode
	char
	Range is {0, 1} 
0: disable.

1: enable.

	Camera.BackLightLevel
	char
	Range is {0,1,2}
0: low
1: middle
2: high

	Camera.R3dnrMode
	char
	Range is {0, 1, 2}
0: disable.
1: auto.
2: manual

	Camera.R3dnrThreshTarget
	unsigned char
	Range is [0 ~ 0xff]

thresh value when manual.

	Camera.DISEnable
	char
	Range is {0, 1}
0: disable.

1: enable.

	Camera.DefogMode
	char
	Range is {0, 1, 2}
0 : disable.

1: auto.

2: manual.

	Camera.DefogStrength
	char
	Range is [0 ~ 100]

Defog strength when used in manual mode.

	Camera.DwdrMode
	char
	Range is {0, 1}
0: disable.

1: enable.

	Camera.DwdrStrength
	unsigned char
	Range is [0 ~ 0xff]

Drdw value when enabled DrdwMode.

	Camera.GainControlMode
	char
	Range is {0, 1, 2, 3}
0: off.

1: low.

2: middle.

3: high

	Camera.WBMode
	char
	Range is {0, 1, 2}
0: auto.

1: manual.

2: indoor.

	Camera.Rgain
	unsigned char
	Range is [0 ~ 0xff].


	Camera.Ggain
	unsigned char
	Range is [0 ~ 0xff].


	Camera.Bgain

	unsigned char
	Range is [0 ~ 0xff].


	Camera.ShutterMode
	char
	Range is {0, 1}.
0: auto.

1: manual.

	Camera.ShutterSpeed
	char
	Range is  [0 ~ 30]
ShutterUsecs value when manual.

Max 1/25 in 50HZ, or max 1/30 in 60HZ.

	Camera.FlickerCtrl
	char
	Range is {0, 1, 2}.
0: 50HZ.

1: 60HZ.

2: disable.


4.3 VideoColor

4.3.1 GetVideoColorConfig
	URL Syntax
	http://<ip>/action/getConfig?name=VideoColor

	Response
	Color.Brightness=50

Color.Contrast=50

Color.Hue=50

Color.Saturation=50

Color.Sharpness=50

	Comment
	ChannelNo = video channel index. Current default channel 0.


4.3.2 SetVideoColorConfig
	URL Syntax
	http://<ip>/action/setConfig?name=VideoColor&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Response
	OK or ERROR

	Comment
	ChannelNo = video channel index. Current default channel 0.


	ParamName
	ParamValue type
	Description

	Color.Brightness
	unsigned char
	Brightness, range is [0 ~ 255]

	Color.Contrast
	unsigned char
	Contrast, range is [0 ~ 255]

	Color.Hue
	unsigned char
	Hue, range is [0 ~ 255]

	Color.Saturation
	unsigned char
	Saturation, range is [0 ~ 255]

	Color.Sharpness
	unsigned char
	Sharpness, range is [0 ~ 255]


4.3.3 Resolution
	Fixed Resolution Name
	Size in PAL
	Size in NTSC

	"D1"
	704 x 576
	704 x 480

	"HD1"
	352 x 576
	352 x 480

	"BCIF"
	704 x 288
	704 x 240

	"CIF"
	352 x 288
	352 x 240

	"QCIF"
	176 x 144
	176 x 120

	"VGA"
	640 x 480

	"QVGA"
	320 x 240

	"SVCD"
	480 x 480

	"QQVGA"
	160 x 128

	"SVGA"
	800 x 592

	"XVGA"
	1024 x 768

	"WXGA"
	1280 x 800

	"SXGA"
	1280 x 1024

	"WSXGA"
	1600 x 1024

	"UXGA"
	1600 x 1200

	"WUXGA"
	1920 x 1200

	"ND1"
	240 x 192

	"720"
	1280 x 720

	"1080"
	1920 x 1080

	"1280x960"
	1280 x 960 (1.3 Mega Pixels)

	"1872x1408"
	1872 x 1408 (2.5 Mega Pixels)

	"3744x1408"
	3744 x 1408 (5 Mega Pixels)

	"2048x1536"
	2048 x 1536 (3 Mega Pixels)

	"2432x2048"
	2432 x 2048 (5 Mega Pixels)

	"1216x1024"
	1216 x 1024 (1.2 Mega Pixels)

	"1408x1024"
	1408 x 1024 (1.5 Mega Pixels)

	"3296x2472"
	3296 x 2472 (8 Mega Pixels)

	"2560x1920"
	2560 x 1920 (5 Mega Pixels)

	"960H",
	960 x 576
	960 x 480

	"DV720P"
	960 x 720


5. Live

5.1 Live General
5.1.1 GetLiveGeneralConfig
	URL Syntax
	http://<ip>/action/getConfig?name=Live

	Comment
	Get the general live config . Current not support.

	Response
	Live.ActionSwitch=1

Live.StreamChoice=0


5.1.2 SetLiveGeneralConfig
	URL Syntax
	http://<ip>/action/setConfig?name=Live&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	Set the general live config. Current not support.

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	Live.ActionSwitch
	char
	Show record or alarm flag in preview window.

	Live.StreamChoice
	char
	Preview stream.

Range is  {0, 1}.

0: main stream.  1: sub stream


5.2 OSD
5.2.1 GetOSDConfig
	URL Syntax
	http://<ip>/action/getConfig?name=OSD

	Comment
	OsdNo : index of OSD.

    0 = osd 1

    1 = osd 2

    2 = osd 3

3 = osd 4

	Response
	Osd[OsdNo].OSDSwitch=1

Osd[OsdNo].OSDType=0

Osd[OsdNo].Alpha=50

Osd[OsdNo].Content=raysharp-ipc

Osd[OsdNo].PosX=20

Osd[OsdNo].PosY=20


5.2.2 SetOSDConfig
	URL Syntax
	http://<ip>/action/setConfig?name=OSD&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	OsdNo : index of OSD. 

    0 = osd 1

    1 = osd 2

    2 = osd 3

    3 = osd 4

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	Osd[OsdNo].OSDSwitch
	char
	Osd switch. 0: disable ; 1: enable.

	Osd[OsdNo].OSDType
	char
	Osd type. 0: time; 1: channel.

	Osd[OsdNo].Alpha
	unsigned char
	Transparency of overlaid osd, the smaller the value, the more transparent it will be.

Range is  [0 ~ 128].

	Osd[OsdNo].Content
	string
	Content of overlaid osd. Max length: 64.

	Osd[OsdNo].PosX
	integer
	Osd postion. Top left x coordinate.

	Osd[OsdNo].PosY
	integer
	Osd postion. Top left y coordinate.


5.3 VideoCover
5.3.1 GetVideoCoverConfig
	URL Syntax
	http://<ip>/action/getConfig?name=VideoCover

	Description
	Video cover config.

	Response
	Cover.Switch=1

Cover[CoReg ].Enable=1

Cover[CoReg ].Rect.X=10
Cover[CoReg ].Rect.Y=30
Cover[CoReg ].Rect.Width=100
Cover[CoReg ].Rect.Height=100

	Comment
	CoReg : Cover Region.
Covers is an array which sustains multi- Cover regions

    0 = region 1

    1 = region 2

    2 = region 3

3 = region 4

head=Cover[CoReg]


5.3.2 SetVideoCoverConfig
	URL Syntax
	http://<ip>/action/setConfig?name=VideoCover&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	CoReg : Cover region index
Covers is an array which contains multiple cover regions

    0 = region 1

    1 = region 2

    2 = region 3

3 = region 4

head    = Cover[CoReg]

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	Cover.Switch
	char
	Video cover switch.

0: disable; 1: enable.

	Cover[CoReg ].Enable
	char
	Each cover area switch. 0: disable; 1: enable.

	Cover[CoReg ].Rect.X
	integer
	top left corner x coordinate (left)

	Cover[CoReg ].Rect.Y
	integer
	top left corner y coordinate (top)

	Cover[CoReg ].Rect.Width 
	integer
	rect width. Range is [0 ~ 65535].

	Cover[CoReg ].Rect.Height
	integer
	rect height. Range is [0 ~ 65535].


5.4 ROI
5.4.1 GetROIConfig
	URL Syntax
	http://<ip>/action/getConfig?name=ROI&type=<StreamType>

	Description
	Stream ROI config. 

	Response
	ROI[RoiNo].Index=5

ROI[RoiNo].Switch=1

ROI[RoiNo].IsAbsQp=1
ROI[RoiNo].Level=6
ROI[RoiNo].BgFrameRate=30
ROI[RoiNo].Rect.X=10
ROI[RoiNo].Rect.Y=30
ROI[RoiNo].Rect.Width=100
ROI[RoiNo].Rect.Height=100

	Comment
	RoiNo : ROI index. Valid value range is [0 ~ 7].

head=ROI[RoiNo ]

StreamType : 0 -- mainstream; 1 -- substream; 2 -- mobilestream.


5.4.2 SetROIConfig
	URL Syntax
	http://<ip>/action/setConfig?name=ROI&type=<StreamType>&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	RoiNo : ROI index. Valid value range is [0 ~ 7].

head=ROI[RoiNo ]

StreamType : 0 -- mainstream; 1 -- substream; 2 -- mobilestream.

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	ROI[RoiNo].Index
	char
	Range is [0 ~ 7].
Roi index, same as priority. 

The larger the index, the higher the priority.

	ROI[RoiNo].Switch
	char
	0: disable.  1: enable.

	ROI[RoiNo].IsAbsQp
	char
	Range is {0, 1}.

Whether to use absolute image quality.

0: relative quality.

1: absolute quality.

	ROI[RoiNo].ILevel
	char
	Range is [1 ~ 6].

Roi value. The larger the value, the better the image quality.

	ROI[RoiNo].BgFrameRate
	char
	Range is [0 ~ 30]

Frame rate of non-ROI region.

	ROI[RoiNo].Rect.X
	integer
	Range is [0 ~ 65535].

top left corner x coordinate (left).

	ROI[RoiNo].Rect.Y
	integer
	Range is [0 ~ 65535].
top left corner y coordinate (top).

	ROI[RoiNo].Rect.Width 
	integer
	rect width.

	ROI[RoiNo].Rect.Height
	integer
	rect height.


6. Stream

6.1 MainStream

6.1.1 GetMainStreamConfig
	URL Syntax
	http://<ip>/action/getConfig?name=MainStream

	Response
	MainStream.VideoType=0
MainStream.BitrateType=0
MainStream.BitrateMode=0
MainStream.Fps=25

MainStream.CustomBitrate=4096
MainStream.Bitrate=3
MainStream.Resolution=1
MainStream.VideoQuality=4

MainStream.IFrameInterval=30
MainStream.LoadBalancing=0
MainStream.AudioSwitch=1

MainStream.ProfileLevel=1


	Comment
	Channel: video channel index. Current default 0.


6.1.2 SetMainStreamConfig
	URL Syntax
	http://<ip>/action/setConfig?name=MainStream&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	Channel: video channel index. Current default 0.



	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	MainStream.VideoType
	char
	Range is {0, 1}.

Video rate type.

0: H.264.

1: H.265.

	MainStream.BitrateType
	char
	Range is {0, 1, 2}.
0: CBR.

1: VBR.

2. FIX QP

	MainStream.BitrateMode
	char
	Range is {0, 1}.
Rate mode.

0: pre define.

1: self define.

	MainStream.Fps
	char
	Range is [1 ~ 25 ]. PAL; 

      or [1 ~ 30] NTSC.
Frames per second.

	MainStream.CustomBitrate
	unsigned integer 
	Range is [1 ~ 16M].
Bitrate value that user can choose.

	MainStream.Resolution
	char
	Range {0, 1, 2, 3, 4, 5}.

0: 300W.

1: 1080P.

2: 960P.

3: 720P.

4: VGA.

5: QVGA.

	MainStream.VideoQuality
	char
	Range {0, 1, 2, 3, 4, 5}.

0: worst.

1: worse.

2: pool.

3: good.

4: better.

5: best.

	MainStream.IFrameInterval
	char
	Range is [1 ~ 100]. unit is second.
I Frame interval.

	MainStream.LoadBalancing
	char
	Range is {0, 1}.
Load balance.

0: real time
1: balanced.

	MainStream.AudioSwitch
	char
	Audio switch.

0: off.  1: on.

	ProfileLevel
	char
	Range is {0, 1, 2}.
Profile level.

0: Baseline.
1: Main Profile.

2: High Profile.




6.2 Sub Stream

6.2.1 GetSubStreamConfig
	URL Syntax
	http://<ip>/action/getConfig?name=SubStream

	Response
	SubStream.VideoType=0
SubStream.BitrateType=0
SubStream.BitrateMode=0
SubStream.Fps=15

SubStream.CustomBitrate=2048
SubStream.Bitrate=3
SubStream.Resolution=1
SubStream.VideoQuality=4

SubStream.IFrameInterval=30
SubStream.LoadBalancing=0
SubStream.AudioSwitch=1

ProfileLevel=1

	Comment
	Channel: video channel index. Current default 0.


6.2.2 SetSubStreamConfig
	URL Syntax
	http://<ip>/action/setConfig?name=SubStream&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	Channel: video channel index. Current default 0.

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	SubStream.VideoType
	char
	Range is {0, 1}.

Video rate type.

0: H.264.

1: H.265.

	SubStream.BitrateType
	char
	Range is {0, 1, 2}.
0: CBR.

1: VBR.

2. FIX QP

	SubStream.BitrateMode
	char
	Range is {0, 1}.
Rate mode.

0: pre define.

1: self define.

	SubStream.Fps
	char
	Range is [1 ~ 25 ]. PAL; 

      or [1 ~ 30] . NTSC.
Frames per second.

	SubStream.CustomBitrate
	unsigned integer 
	Range is [1 ~ 16M].
Bitrate value that user can choose.

	SubStream.Resolution
	char
	Range {0, 1, 2, 3, 4, 5}.

0: 300W.

1: 1080P.

2: 960P.

3: 720P.

4: VGA.

5: QVGA.

	SubStream.VideoQuality
	char
	Range {0, 1, 2, 3, 4, 5}.

0: worst.

1: worse.

2: pool.

3: good.

4: better.

5: best.

	SubStream.IFrameInterval
	char
	Range is [1 ~ 100]. unit is second.
I Frame interval.

	SubStream.LoadBalancing
	char
	Range is {0, 1}.
Load balance.

0: real time
1: balanced.

	SubStream.AudioSwitch
	char
	Audio switch.

0: off.  1: on.

	SubStream.ProfileLevel
	char
	Range is {0, 1, 2}.
Profile level.

0: Baseline.
1: Main Profile.

2: High Profile.




6.3 Mobile Stream

6.3.1 GetMobileStreamConfig
	URL Syntax
	http://<ip>/action/getConfig?name=MobileStream

	Response
	MobileStream.VideoSwitch=1

MobileStream.VideoType=0
MobileStream.BitrateType=0
MobileStream.BitrateMode=0
MobileStream.Fps=10
MobileStream.CustomBitrate=512
MobileStream.Bitrate=5
MobileStream.Resolution=1
MobileStream.VideoQuality=4

MobileStream.IFrameInterval=30
MobileStream.LoadBalancing=0
MobileStream.AudioSwitch=1

MobileStream.ProfileLevel=1

	Comment
	Channel: video channel index. Current default 0.


6.3.2 SetMobileStreamConfig
	URL Syntax
	http://<ip>/action/setConfig?name=MobileStream&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	Channel: video channel index. Current default 0.

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	MobileStream.VideoSwitch
	char
	Video switch.

0: disabled. 1: enabled.

	MobileStream.VideoType
	char
	Range is {0, 1}.

Video rate type.

0: H.264.

1: H.265.

	MobileStream.BitrateType
	char
	Range is {0, 1, 2}.
0: CBR.

1: VBR.

2. FIX QP

	MobileStream.BitrateMode
	char
	Range is {0, 1}.
Rate mode.

0: pre define.

1: self define.

	MobileStream.Fps
	char
	Range is [1 ~ 25 ]. PAL; 

      or [1 ~ 30] . NTSC.
Frames per second.

	MobileStream.CustomBitrate
	unsigned integer 
	Range is [1 ~ 16M].
Bitrate value that user can choose.

	MobileStream.Resolution
	char
	Range {0, 1, 2, 3, 4, 5}.

0: 300W.

1: 1080P.

2: 960P.

3: 720P.

4: VGA.

5: QVGA.

	MobileStream.VideoQuality
	char
	Range {0, 1, 2, 3, 4, 5}.

0: worst.

1: worse.

2: pool.

3: good.

4: better.

5: best.

	MobileStream.IFrameInterval
	char
	Range is [1 ~ 100]. unit is second.
I Frame interval.

	MobileStream.LoadBalancing
	char
	Range is {0, 1}.
Load balance.

0: real time
1: balanced.

	MobileStream.AudioSwitch
	char
	Audio switch.

0: off.  1: on.

	MobileStream.ProfileLevel
	char
	Range is {0, 1, 2}.
Profile level.

0: Baseline.
1: Main Profile.

2: High Profile.


6.4 Audio
6.4.1 GetAudioConfig
	URL Syntax
	http://<ip>/action/getSysConfig?name=Audio

	Response
	Audio.Switch=1

Audio.Type=0
Audio.Bitrate=0

Audio.MicInVolume=5
Audio.LineOutVolume=6

	Comment
	Channel: video channel index. Current default 0.


6.4.2 SetAudioConfig
	URL Syntax
	http://<ip>/action/setSysConfig?name=Audio&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	Channel: video channel index. Current default 0.

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	Audio.Switch
	unsigned char
	Audio switch.

0: off.  1: on.

	Audio.Type
	char
	Range is {0, 1, 2, 3}.
Audio type.

0: G711A; 1: G711U.

2: G726;  3: ADPCM.

	Audio.Bitrate
	unsigned char
	Current set to 0 which means 64k.

	Audio.MicInVolume
	unsigned char
	Range is [0 ~ 10].
In volume value.

	Audio.LineOutVolume
	unsigned char
	Range is [0 ~ 10].
Out volume value.


7. NetWork

7.1 BasicConfig

7.1.1 GetBasicConfig

	URL Syntax
	http://<ip>/action/getNetConfig?name=NetBase

	Comment
	Basic config contains basic network parameters (Default interface, domain name, host name), and configuration of each network interface.

	Response
	NetBase.IPAddress=192.168.1.240
NetBase.Netmask=255.255.255.0
NetBase.Gateway=192.168.1.1

NetBase.Dns1=8.8.8.8

NetBase.Dns2=221.96.128.86

NetBase.PPPOEuser=raysharp
NetBase.Password=519070
NetBase.NetworkMode=2
NetBase.ClientPort=9001
NetBase.WebPort=80
NetBase.MobilePort=18040

NetBase.UseUPNP=0


7.1.2 SetBasicConfig

	URL Syntax
	http://<ip>/action/setNetConfig?name=NetBase&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	NetBase.IPAddress
	string
	Interface IP address. Max length: 32.

	NetBase.Netmask
	string
	Network mask string: 
In the form of x.x.x.x, range of x is [0-255]

Example:
255.255.255.0

	NetBase.Gateway
	string
	Gateway address. 

	NetBase.Dns1
	string
	IP address of first DNS server. 

	NetBase.Dns2
	string
	IP address of second DNS server. 

	NetBase.PPPOEuser
	string
	PPPoE User name. Max length: 32.

	NetBase.Password
	string
	PPPoE User password. Max length: 32.

	NetBase.NetworkMode
	char
	Range is {0, 1, 2}.
Networking mode.

0: DHCP.

1: PPPOE.

2: manual.

	NetBase.ClientPort
	unsigned short
	Client port.

	NetBase.WebPort
	unsigned short
	Web port.

	NetBase.MobilePort
	unsigned short
	Mobile port.

	NetBase.UseUPNP
	char
	Enable / disable UPnP.

0: disable. 1: enable.


7.2 3G
7.2.1 Get3GConfig
	URL Syntax
	http://<ip>/action/getNetConfig?name=Net3G

	Comment
	Current not support.

	Response
	Net3G.Apn=ChinaNet
Net3G.DataServer=123456

Net3G.UserName=raysharp

Net3G.Password=519070

Net3G.NetworkType=WCDMA


7.2.2 Set3GConfig
	URL Syntax
	http://<ip>/action/setNetConfig?name=Net3G&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	Current not support.

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	Net3G.Apn
	string
	Data service access point. Max length: 24.

	Net3G.DataServer
	string
	Data center number. Max length: 24.

	Net3G.UserName
	string
	user name. Max length: 16.

	Net3G.Password
	string
	User password. Max length: 16.

	Net3G.NetworkType
	char
	Range is {1, 2, 3}.

1: WCDMA.

2: EVD0 

3: ...


7.3 Wifi
7.3.1 GetWifiConfig
	URL Syntax
	http://<ip>/action/getNetConfig?name=Wifi

	Comment
	Current not support.

	Response
	Wifi.SSID=raysharp
Wifi.Password=519070
Wifi.IPAddress=192.168.1.114
Wifi.Gateway=192.168.1.1
Wifi.Netmask=255.255.255.0
Wifi.DNS=202.96.128.86
Wifi.EncryptType=WPA-PSK
Wifi.AuthMode=AES
Wifi.Switch=1


7.3.2 SetWifiConfig
	URL Syntax
	http://<ip>/action/setNetConfig?name=Wifi&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	Current not support.

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	Wifi.SSID
	string
	wifi hotspot. Max length: 32.

	Wifi.Password 
	string
	Wifi password. Max length: 32.

	Wifi.IPAddress
	string
	IP address. Max length: 32.

	Wifi.Gateway
	string
	Gateway of wifi. 

	Wifi.Netmask 
	string
	Subnet mask. 

	Wifi.DNS
	string
	Domain server.

	Wifi.EncryptType
	char
	Range is {0, 1, 2, 3, 4}.

0: NONE.

1: WEP.

2: WPA-PSK.

3: WPA2-PSK.

4: WPA-NONE.

	Wifi.AuthMode
	char
	Range is {1, 2, 3, 4}.

Authentication Mode.

1: OPEN.

2: SHARED.

3: TKIP.

4: AES.

	Wifi.Switch
	char
	0: close.  1: open.


7.4 SSID
7.4.1 GetSSIDConfig
	URL Syntax
	http://<ip>/action/getNetConfig?name=SSID

	Comment
	SSIDNo : index of recorded wifi hotspot. Valid value range is [0 ~ 4]. Current not support.

	Response
	SSID[SSIDNo ].SSID=raysharp-free
SSID[SSIDNo ].Protocol=802.11

SSID[SSIDNo ].EncryptType=WPA-PSK
SSID[SSIDNo ].AuthMode=AES
SSID[SSIDNo ].SignalQuality=5

SSID[SSIDNo ].Frequency=50

SSID[SSIDNo ].MacAddr=00:23:5c:f2:1c:b4


7.4.2 SetSSIDConfig
	URL Syntax
	http://<ip>/action/setNetConfig?name=SSID&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	SSIDNo : index of recorded wifi hotspot. Valid value range is [0 ~ 4]. Current not support.

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	SSID[SSIDNo ].SSID
	string
	Once connected wifi hotspot.

Max length: 32.

	SSID[SSIDNo ].Protocol
	string
	802.11. Max length: 16.

	SSID[SSIDNo ].EncryptType
	char
	Range is {0, 1, 2, 3, 4}.

0: NONE.

1: WEP.

2: WPA-PSK.

3: WPA2-PSK.

4: WPA-NONE.

	SSID[SSIDNo ].AuthMode
	char
	Range is {1, 2, 3, 4}.

Authentication Mode.

1: OPEN.

2: SHARED.

3: TKIP.

4: AES.

	SSID[SSIDNo ].SignalQuality
	char
	Range is [0 ~ 5].

0: lowest quality.
5: highest quality.

	SSID[SSIDNo ].Frequency
	unsigned char
	Range is [1 ~ 255]

	SSID[SSIDNo ].MacAddr
	string
	MAC address of wifi ssid. Max length: 6.


7.5 DDNS

7.5.1 GetDDNSConfig
	URL Syntax
	http://<ip>/action/getNetConfig?name=DDNS

	Comment
	

	Response
	DDNS.HostName=raysharp
DDNS.ServerAddr=anlian.co
DDNS.UserName=raysharp
DDNS.Password=519070
DDNS.UseFlag=1
DDNS.Switch=1
DDNS.UpdateInterval=3


7.5.2 SetDDNSConfig
	URL Syntax
	http://<ip>/action/setNetConfig?name=DDNS&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	DDNS.HostName
	string
	Host name of this device. Max length: 32.

	DDNS.ServerAddr
	string
	DDNS server IP address or name. Max length: 32.

	DDNS.UserName
	string
	DDNS user name. Max length: 32.

	DDNS.Password
	string
	DDNS user password. Max length: 32.

	DDNS.UseFlag
	unsigned integer
	DDNS use flag.

Each bit correspond to a type of DDNS.

	DDNS.Switch
	unsigned char 
	DDSN switch.

0: disable.  1: enable.

	DDNS.UpdateInterval
	unsigned char 
	Range is [0 ~ 255].

DDNS update interval.


7.6 Email

7.6.1 GetEmailConfig
	URL Syntax
	http://<ip>/action/getNetConfig?name=Email

	Comment
	

	Response
	Email.Address=www.163.com

Email.Sender=www.raysharp.cn

Email.Password=123456

Email.Receivers[0]=x@raysharp.cn

Email.Receivers[1]=y@raysharp.cn

Email.Receivers[2]=z@raysharp.cn

Email.Port=26

Email.SSLSwitch=1

Email.Switch=1

Email.Intervaltime=180


7.6.2 SetEmailConfig
	URL Syntax
	http://<ip>/action/setNetConfig?name=Email&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	Email.Address
	string
	SMTP server IP address or name.

Max length: 32.

	Email.Sender
	string
	Sender address. Max length: 32.

	Email.Password
	string
	Sender password. Max length: 32.

	Email.Receivers[0]
	string
	Email addresses of 3 receivers.

Max length: 32.

	Email.Receivers[1]
	
	

	Email.Receivers[2]
	
	

	Email.Port
	unsigned short
	Range is [1-65535].

	Email.SSLSwitch
	char
	1: enable SSL email.

	Email.EmailSwitch
	char
	0: disable.  1: enable email.

	Email.Intervaltime
	short
	Range is {180, 300, 600}.

unit is second.


7.7 P2P
7.7.1 GetP2PConfig
	URL Syntax
	http://<ip>/action/getNetConfig?name=P2P

	Comment
	

	Response
	P2P.Uid=rs123456
P2P.UserName=raysharp
P2P.Password=519070
P2P.State=1


7.7.2 SetP2PConfig
	URL Syntax
	http://<ip>/action/setNetConfig?name=P2P&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	P2P.Uid
	string
	P2P id. Max length: 32.

	P2P.UserName
	string
	P2P user name. Max length: 32.

	P2P.Password
	string
	P2P user password. Max length: 32.

	P2P.State
	char
	P2P connect status.


7.8 RTSP
7.8.1 GetRTSPConfig
	URL Syntax
	http://<ip>/action/getNetConfig?name=RTSP

	Comment
	

	Response
	RTSP.UserName=raysharp
RTSP.Password=519070
RTSP.Port=554

RTSP.CheckFlag=1

RTSP.Switch=1

RTSP.AuthType=1


7.8.2 SetRTSPConfig
	URL Syntax
	http://<ip>/action=setNetConfig?name=RTSP&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	RTSP.UserName
	string
	RTSP user name. Max length: 32.

	RTSP.Password
	string
	RTSP user password. Max length: 32.

	RTSP.Port
	integer
	Range is [1-65535].
Port of RTSP server.

	RTSP.CheckFlag
	char
	0: disable.  1: enable

	RTSP.Switch
	char
	0: disable.  1: enable.

	RTSP.AuthType
	char
	Range is {0, 1, 2}.

0: none.

1: basic.

2. digest.


7.9 FTP
7.9.1 GetFTPConfig
	URL Syntax
	http://<ip>/action/getNetConfig?name=FTP

	Comment
	Return FTP server config.

	Response
	FTP.IPAddress=192.168.1.120
FTP.UserName=anonymity

FTP.Password=123456

FTP.Port =21

FTP.Switch=1

FTP.TimeInterval = 3
FTP.ReqTypeMask =2
FTP.RecType =1
FTP.FilePath=share


7.9.2 SetFTPConfig
	URL Syntax
	http://<ip>/action/setNetConfig?name=FTP&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	FTP.IPAddress
	string
	FTP IP address. Max length: 32.

	FTP.UserName
	string
	Login user name. Max length: 32.

	FTP.Password 
	string
	Login user password. Max length: 32.

	FTP.Port
	integer
	FTP port.

	FTP.Switch
	char
	0: off.  1: on.

	FTP.TimeInterval 
	char
	Range is [0 ~ 128].
Period of time to upload video.

	FTP.ReqTypeMask 
	char
	Req type mask .Bitwise representation.

0 bit: close.

2bit: image.

3bit: video

	FTP.RecType 
	char
	Range is {1, 2, 3}.

1: normal record.

2: alarm record.

3: all record.

	FTP.FilePath
	string
	FTP file path. Max length: 96.
Directory name.


7.10 Platform
7.10.1 GetPlatformConfig
	URL Syntax
	http://<ip>/action/getNetConfig?name=Platform 

	Comment
	Current not support.

	Response
	Platform.PuID=rs123456puid

Platform.IPAddress=192.168.1.245

Platform.ShowTag=1

Platform.Port=3456

Platform.Select=1

Platform.Switch=1

Platform.MediaConType=0


7.10.2 SetPlatformConfig
	URL Syntax
	http://<ip>/action/setNetConfig?name=Platform&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	 Current not support.

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	Platform.PuID
	string
	Platform PUID. Max length: 32.

	Platform.IPAddress
	string
	Platform IP address. Max length: 32.

	Platform.ShowTag
	integer
	Show tag.

	Platform.Port
	unsigned short
	Platform port.

	Platform.Select
	char
	Platform select.

	Platform.Switch
	char
	0: disable.  1: enable.

	Platform.MediaConType
	char
	Server transfer protocol.

0: TCP.

1: UDP. 


7.11 Mobile
7.11.1 GetMobileConfig
	URL Syntax
	http://<ip>/action/getNetConfig?name=Mobile

	Comment
	Current not support.

	Response
	Mobile.UserName=raysharp

Mobile.Password=519070

Mobile.IPAddress=192.168.1.10

Mobile.NetType=3G


7.11.2 SetMobileConfig
	URL Syntax
	http://<ip>/action/setNetConfig?name=Mobile&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	Current not support.

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	Mobile.UserName
	string
	Mobile user name. Max length: 32.

	Mobile.Password
	string
	Mobile user password. Max length: 32.

	Mobile.IPAddress
	string
	Mobile IP Address. Max length: 32.

	Mobile.NetType
	char
	Networking type.

0: 2.5G.

1: 2.75G.

2: 3G.

3: Wifi.


7.12 GB28181
7.12.1 GetGB28181Config
	URL Syntax
	http://<ip>/action/getNetConfig?name=GB28181

	Comment
	Current not support.

	Response
	GB28181.DeviceId=3556
GB28181.AlarmId=123
GB28181.SipServerId=37777

GB28181.SipServerIP=192.168.1.129

GB28181.SipServerPwd=519070

GB28181.SipServerPort=5566

GB28181.Switch=1


7.12.2 SetGB28181Config
	URL Syntax
	http://<ip>/action=setNetConfig?name=GB28181&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	Current not support.

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	GB28181.DeviceId
	string
	Device ID. Max length: 64.

	GB28181.AlarmId
	string
	Alarm ID. Max length: 64.

	GB28181.SipServerId
	string
	SIP Server ID. Max length: 32.

	GB28181.SipServerIP
	string
	SIP Server IP. Max length: 32.

	GB28181.SipServerPwd
	string
	SIP Server password. Max length: 64.

	GB28181.SipServerPort
	unsigned short
	SIP Server port.

	GB28181.Switch
	char
	0: disable.  1: enable.


8. Alarm
8.1 IO Alarm

8.1.1 GetIOAlarmConfig
	URL Syntax
	http://<ip>/action/getAlarmConfig?name=IOAlarm

	Comment
	

	Response
	IOAlarm.AlarmSet=1
IOAlarm.AlarmOutTime=10
IOAlarm.RecordDelayTime=30
IOAlarm.AlarmOutManager=1

IOAlarm.EmailLink=0

IOAlarm.RecordSwitch=1

IOAlarm.FtpUploadSwitch=0

IOAlarm.CloudSwitch=0

IOAlarm.PushSwitch=0


8.1.2 SetIOAlarmConfig
	URL Syntax
	http://<ip>/action/setAlarmConfig?name=IOAlarm&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	IOAlarm.AlarmSet
	char
	Range is {0, 1, 2}.

IO status alarm.

0: normal open.

1: normal close.

2: closed.

	IOAlarm.AlarmOutTime
	char
	Range is 10, 20, 40, 60, 30s, 1min , 2min.

	IOAlarm.RecordDelayTime
	char
	Range is 10, 30, 1min, 2min, 5mins.

	IOAlarm.AlarmOutManager
	char
	Alarm out switch. If only one alarm out, just use AlarmOutManager[0]. Four alarm out supported.

	IOAlarm.EmailLink
	char
	Email link switch.

0: disable.  1: enable.

	IOAlarm.RecordSwitch
	char
	0: disable.  1: enable.

	IOAlarm.FtpUploadSwitch
	char
	0: disable.  1: enable.

	IOAlarm.CloudSwitch
	char
	0: disable.  1: enable.

	IOAlarm.PushSwitch
	char
	0: disable.  1: enable.


8.2 MotionDetect

8.2.1 GetMotionDetectConfig
	URL Syntax
	http://<ip>/action/getAlarmConfig?name=Motion

	Comment
	MotionDetect config of a video channel contains Enable, Level, Region and so on.

	Response
	Motion.Switch=1

Motion.Sensitivity=4

Motion.AlarmOutTime=10
Motion.RecordDelayTime=30
Motion.AlarmOutManager=1

Motion.EmailLink=0

Motion.RecordSwitch=1

Motion.FtpUploadSwitch=0

Motion.CloudSwitch=0

Motion.PushSwitch=0

Motion.Region[0]=03-CD-5E-7F-12-6A-49-FB-6C-10-2D

...

...

Motion.Region[17]=4B-AF-71-CE-38-90-A6-CA-DF-33-5C


8.2.2 SetMotionDetectConfig
	URL Syntax
	http://<ip>/action/setAlarmConfig?name=Motion&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	Channel: video channel index
LineNum

Index of region, region is divided into lines and each line has several blocks, a line is divided into 44 bit , a bit for a block..
  0=Line 1

  1=Line 2

  …

  …

17= Line 18


	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	Motion.Switch
	char
	Enable/Disable motion detect feature in a channel.

	Motion.Sensitivity
	char
	Range is [1-8].
Sensitivity of motion detection.
1: lowest sensitivity.
8: highest sensitivity.

	Motion.Region[LineNum]
	integer
	Currently, region is divided into 36 lines and 44 blocks/line.
A bit describes a block in the line.
Bit = 1: motion in this block is monitored..
Example: 
Region[0] = (FF-FF-FF-FF-FF-F0-00-00-00-00-00):: motion in channel 0 line 0’s 44 blocks is monitored. Line 1’s 44 blocks not monitored.


	Motion.AlarmOutTime
	char
	Range is 10, 20, 40, 60, 30s, 1min , 2min.

	Motion.RecordDelayTime
	char
	Range is 10, 30, 1min, 2min, 5mins.

	Motion.AlarmOutManager
	char
	Alarm out switch. If only one alarm out, just use AlarmOutManager[0]. Four alarm out supported.

	Motion.EmailLink
	char
	Email link switch.

0: disable.  1: enable.

	Motion.RecordSwitch
	char
	0: disable.  1: enable.

	Motion.FtpUploadSwitch
	char
	0: disable.  1: enable.

	Motion.CloudSwitch
	char
	0: disable.  1: enable.

	Motion.PushSwitch
	char
	0: disable.  1: enable.


8.3 PerimeterLine
8.3.1 GetPerimeterLineConfig
	URL Syntax
	http://<ip>/action/getAlarmConfig?name=PerimLine

	Comment
	LineNo below is the perimeter line index. Valid value range is [0 ~ 31]. 

	Response
	PerimLine.ChnSwitch=1

PerimLine.AlarmOutTime=10
PerimLine.RecordDelayTime=30
PerimLine.AlarmOutManager=1

PerimLine.EmailLink=0

PerimLine.RecordSwitch=1

PerimLine.FtpUploadSwitch=0

PerimLine.CloudSwitch=0

PerimLine.PushSwitch=0

PerimLine.RuleSwitch=1

PerimLine.RuleType=1

PerimLine.LineTwoWay=1

PerimLine.Scene=1

PerimLine.Sensitive=3

PerimLine.RuleLine[LineNo ].StartX=10

PerimLine.RuleLine[LineNo ].StartY=10

PerimLine.RuleLine[LineNo ].EndX=20

PerimLine.RuleLine[LineNo ].EndY=30


8.3.2 SetPerimeterLineConfig
	URL Syntax
	http://<ip>/action/setAlarmConfig?name=PerimLine&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	LineNo below is the perimeter line index. Valid value range is [0 ~ 31]. 
Each channel has at most 64 points coordinates. ie 32 lines. First line will be used if it’s satellite, if it’s perimeter, 8 lines will be connected together to form a closed figure.

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	PerimLine.ChnSwitch
	char
	0: close.  1: open.

	PerimLine.AlarmOutTime
	char
	Range is {0, 1, 2, 3, 4}.

Successively represent : 0, 10, 20, 40, 60.

	PerimLine.RecordDelayTime
	char
	Range is {0, 1, 2, 3, 4}.

Successively represent : 0, 10, 20, 40, 60.

	PerimLine.AlarmOutManager
	char
	Alarm out switch. If only one alarm out, just use AlarmOutManager[0]. Four alarm out supported.

	PerimLine.EmailLink
	char
	Email link switch.

0: disable.  1: enable.

	PerimLine.RecordSwitch
	char
	0: disable.  1: enable.

	PerimLine.FtpUploadSwitch
	char
	0: disable.  1: enable.

	PerimLine.CloudSwitch
	char
	0: disable.  1: enable.

	PerimLine.PushSwitch
	char
	0: disable.  1: enable.

	PerimLine.RuleSwitch
	char
	Each channel use one byte to represent at most 4 rules. 0 - close; 1 - in use.

	PerimLine.RuleType
	char
	Each channel use one byte to represent at most 4 rules type.

 0 - satellite line; 1 - perimeter line.

	PerimLine.LineTwoWay
	char
	Each channel use one byte to represent at most 4 satellite type.

 0 - not two way; 1 - two way.

	PerimLine.Scene
	char
	Scene. 0: indoor, 1: outdoor.

	PerimLine.Sensitive
	char
	Sensitive. Range is {0, 1,  2, 3}.

	PerimLine.RuleLine[LineNo ].StartX
	short
	Start x coordinate of perimeter line.

	PerimLine.RuleLine[LineNo ].StartY
	short
	Start y coordinate of perimeter line.

	PerimLine.RuleLine[LineNo ].EndX
	short
	End x coordinate of perimeter line.

	PerimLine.RuleLine[LineNo ].EndY
	short
	End y coordinate of perimeter line.


8.4 OSCAlarm
8.4.1 GetOSCAlarm
	URL Syntax
	http://<ip>/action/getAlarmConfig?name=OSCAlarm

	Comment
	RectNo below is the OSC rect index. Valid value range is [0 ~ 15]. 

	Response
	OSCAlarm.ChnSwitch=1

OSCAlarm.AlarmOutTime=10
OSCAlarm.RecordDelayTime=30
OSCAlarm.AlarmOutManager=1

OSCAlarm.EmailLink=0

OSCAlarm.RecordSwitch=1

OSCAlarm.FtpUploadSwitch=0

OSCAlarm.CloudSwitch=0

OSCAlarm.PushSwitch=0

OSCAlarm.RuleSwitch=1

OSCAlarm.RuleType=1

OSCAlarm.RuleRectEnable[0]=1

OSCAlarm.RuleRectEnable[1]=1

OSCAlarm.RuleRectEnable[2]=1

OSCAlarm.RuleRectEnable[3]=1

OSCAlarm.Scene=1

OSCAlarm.Sensitive=2

OSCAlarm.Rect[RectNo ].X=10

OSCAlarm.Rect[RectNo ].Y=10

OSCAlarm.Rect[RectNo ].Width=50

OSCAlarm.Rect[RectNo ].Height=50


8.4.2 SetOSCAlarm
	URL Syntax
	http://<ip>/action/setAlarmConfig?name=OSCAlarm

	Comment
	RectNo below is the OSC rect index. Valid value range is [0 ~ 15]. 
Total regions is at most 16. 0 ~ 3 is used for the first rule; 4 ~ 7 is used for the second rule; 8 ~ 11 is used for the third rule; 12 ~ 15 is used for the forth rule.

	Response
	result=2


	ParamName
	ParamValue type
	Description

	OSCAlarm.ChnSwitch
	char
	0: close.  1: open.

	OSCAlarm.AlarmOutTime
	char
	Range is {0, 1, 2, 3, 4}.

Successively represent : 0, 10, 20, 40, 60.

	OSCAlarm.RecordDelayTime
	char
	Range is {0, 1, 2, 3, 4}.

Successively represent : 0, 10, 20, 40, 60.

	OSCAlarm.AlarmOutManager
	char
	Alarm out switch. If only one alarm out, just use AlarmOutManager[0]. Four alarm out supported.

	OSCAlarm.EmailLink
	char
	Email link switch.

0: disable.  1: enable.

	OSCAlarm.RecordSwitch
	char
	0: disable.  1: enable.

	OSCAlarm.FtpUploadSwitch
	char
	0: disable.  1: enable.

	OSCAlarm.CloudSwitch
	char
	0: disable.  1: enable.

	OSCAlarm.PushSwitch
	char
	0: disable.  1: enable.

	OSCAlarm.RuleSwitch
	char
	Each channel use one byte to represent at most 4 rules. 0 - close; 1 - in use.

	OSCAlarm.RuleType
	char
	Each channel use one byte to represent at most 4 rules type.

 0 - left; 1 - lost.

	OSCAlarm.RuleRectEnable[0]
	char
	Each channel has at most 4 rules, each rule has at most 4 regions, each bit represent a status of a region. 0 - disable; 1 - enable.

	OSCAlarm.RuleRectEnable[1]
	
	

	OSCAlarm.RuleRectEnable[2]
	
	

	OSCAlarm.RuleRectEnable[3]
	
	

	OSCAlarm.Scene
	char
	Scene. 0: indoor, 1: outdoor.

	OSCAlarm.Sensitive
	char
	Sensitive. Range is {0, 1,  2, 3}.

	OSCAlarm.Rect[RectNo ].X
	integer
	Left coordinate of rule rect.

	OSCAlarm.Rect[RectNo ].Y
	integer
	Top coordinate of rule rect.

	OSCAlarm.Rect[RectNo ].Width
	integer
	Width of rule rect.

	OSCAlarm.Rect[RectNo ].Height
	integer
	Height of rule rect.


8.5 ODAlarm
8.5.1 GetODAlarm
	URL Syntax
	http://<ip>/action/getAlarmConfig?name=ODAlarm

	Comment
	

	Response
	ODAlarm.Switch=1

ODAlarm.Sensitivity=3

ODAlarm.AlarmOutTime=10
ODAlarm.RecordDelayTime=30
ODAlarm.AlarmOutManager=1

ODAlarm.EmailLink=0

ODAlarm.RecordSwitch=1

ODAlarm.FtpUploadSwitch=0

ODAlarm.CloudSwitch=0

ODAlarm.PushSwitch=0


8.5.2 SetODAlarm
	URL Syntax
	http://<ip>/action/setAlarmConfig?name=ODAlarm

	Comment
	

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	ODAlarm.Switch
	char
	0: disable.  1: enable.

	ODAlarm.Sensitivity
	char
	Range is [1 ~ 6].

Sensitivity of OD.
1: lowest sensitivity.
6: highest sensitivity.

	ODAlarm.AlarmOutTime
	char
	Range is 10, 20, 40, 60, 30s, 1min , 2min.

	ODAlarm.RecordDelayTime
	char
	Range is 10, 30, 1min, 2min, 5mins.

	ODAlarm.AlarmOutManager
	char
	Alarm out switch. If only one alarm out, just use AlarmOutManager[0]. Four alarm out supported.

	ODAlarm.EmailLink
	char
	Email link switch.

0: disable.  1: enable.

	ODAlarm.RecordSwitch
	char
	0: disable.  1: enable.

	ODAlarm.FtpUploadSwitch
	char
	0: disable.  1: enable.

	ODAlarm.CloudSwitch
	char
	0: disable.  1: enable.

	ODAlarm.PushSwitch
	char
	0: disable.  1: enable.


9. Record & Snap

9.1 General Record

9.1.1 GetGeneralRecConfig
	URL Syntax
	http://<ip>/action/getConfig?name=GeneralRec

	Comment
	Current not support.

	Response
	Record.ChnSwitch=1

Record.PreRecSwitch=1

Record.State=1

Record.Mode=1

Record.StreamMode=0


9.1.2 SetGeneralRecConfig
	URL Syntax
	http://<ip>/action/setConfig?name=GeneralRec&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	Current not support.

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	Record.ChnSwitch
	char
	0: close.  1: open.

	Record.PreRecSwitch
	char
	0: close.  1: open.

	Record.State
	char
	Record automatically restored when power off restart.

	Record.Mode
	char
	0: Timer recording.

1: Boot recording.

	Record.StreamMode
	char
	0: mainstream. 1: substream.


9.2 Smart Record
9.2.1 GetSmartRecConfig
	URL Syntax
	http://<ip>/action/getConfig?name=SmartRec

	Comment
	Current not support.

	Response
	SmartRec.Type=0


9.2.2 SetSmartRecConfig
	URL Syntax
	http://<ip>/action=setConfig?name=SmartRec&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	channel in below table is video channel index, start form 0. Current not support.

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	SmartRec.Type
	char
	Range is {0, 1}.
0: motion record

1: smart record


9.3 Schedule
9.3.1 GetScheduleConfig
	URL Syntax
	http://<ip>/action/getConfig?name=Schedule

	Comment
	Channel in below table is video channel number, weekday range is [0-6] (Sunday - Saturday).

Time range is [0 ~ 1]. Half hour as a unit. 

	Response
	Schedule[0][0]=FFFFFFFFFFFFFFFFFFFFFFFF

Schedule[0][1]=FFFFFFFFFFFFFFFFFFFFFFFF

Schedule[1][0]=FFFFFFFFFFFFFFFFFFFFFFFF

Schedule[1][1]=FFFFFFFFFFFFFFFFFFFFFFFF

...

Schedule[6][0]=000000000000000000000000

Schedule[6][1]=000000000000000000000000


9.3.2 SetScheduleConfig
	URL Syntax
	http://<ip>/action=setConfig?name=Schedule&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	In below table: ch = channel index, wd = week day index, ts = time section index. 

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	Schedule[wd][ts]
	char
	wd (week day) range is [0 ~ 6] (Sunday - Staurday)

ts (time section) range is [0 ~ 1], timesection table index.
Bit 1: normal record schedule.
Bit 2: motion record schedule.

Bit 3: IO alarm record schedule.

Bit 4: intelligence record schedule.


9.4 Snap

9.4.1 GetSnapConfig
	URL Syntax
	http://<ip>/action/getConfig?name=Snap

	Comment
	Channel in below table is video channel number. Current not support.

	Response
	Snap.Switch=1

Snap.Type=2

Snap.Interval=20

Snap.Quality=1


9.4.2 SetSnapConfig
	URL Syntax
	http://<ip>/action/setConfig?name=Snap&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	Current not support.

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	Snap.Switch
	char
	0: disable.  1: enable.

	Snap.Type
	char
	Bitwise value.

0: motion.

1: IO.

2: smart.

	Snap.Interval
	char
	Range is [1 ~ 100].

	Snap.Quality
	char
	Image quality of capture.


10. System

10.1 General

10.1.1 GetSysGeneralConfig
	URL Syntax
	http://<ip>/action/getSystemConfig?name=SystemInfo

	Comment
	

	Response
	System.LangUseFlag=1

System.CurrentLang=1

System.VideoFormat=1

System.DateMode=0

System.TimeMode=0

System.Time=2015-06-18_15:03:00

System.MaintainSwitch=1

System.MaintainPeriod1=0

System.MaintainPeriod2=1

System.MaintainTime=03:00


10.1.2 SetSysGeneralConfig
	URL Syntax
	http://<ip>/action/setSystemConfig?name=SystemInfo&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	System.LangUseFlag
	integer
	Language flag of current usage, one bit represent a kind of language.

	System.CurrentLang
	char
	Current system language.

	System.VideoFormat
	char
	0 - TS_PAL, 1 - TS_NTSC

	System.DateMode
	char
	Date mode

	System.TimeMode
	char
	Time mode.

0: 24 hours mode.

1: 12 hours mode.

	System.Time
	string
	System time . 

Format: Year-month-Day_Hour:minute:Second .

	System.MaintainSwitch
	char
	0: close.  1: open.

	System.MaintainPeriod1
	char
	0: every day.

1: every week.

2: every month.

	System.MaintainPeriod2
	char
	Some day in a week or some day in a month.

	System.MaintainTime
	string
	Maintain time . 

Format: Hour:minute .


10.2 User

10.2.1 GetUserInfo
	URL Syntax
	http://<ip>/action/getSystemConfig?name=UserInfo&index=<UserNo>

	Comment
	UserNo: user list index. Valid value range is [0 ~ 6]. 0 : admin

	Response
	User[UserNo].UserName=raysharp

User[UserNo].Password=519070

User[UserNo].PwdConfirm=519070

User[UserNo].SetRight=1

User[UserNo].PreviewChannel=1

User[UserNo].PlayBackChannel=1

User[UserNo].BackupChannel=1

User[UserNo].PlayBack=1

User[UserNo].Backup=1

User[UserNo].Preview=1

User[UserNo].Switch=1

User[UserNo].PwdSwitch=0

User[UserNo].IsLive=1


10.2.2 GetUserInfoAll
	URL Syntax
	http://<ip>/action/getSystemConfig?name=UserInfoAll

	Comment
	UserNo: user list index. Valid value range is [0 ~ 6].

	Response
	User[UserNo].UserName=raysharp

User[UserNo].Password=519070

User[UserNo].PwdConfirm=519070

User[UserNo].SetRight=1

User[UserNo].PreviewChannel=1

User[UserNo].PlayBackChannel=1

User[UserNo].BackupChannel=1

User[UserNo].PlayBack=1

User[UserNo].Backup=1

User[UserNo].Preview=1

User[UserNo].Switch=1

User[UserNo].PwdSwitch=0

User[UserNo].IsLive=1


10.2.3 SetUserConfig
	URL Syntax
	http://<ip>/action/setSystemConfig?name=UserInfo

	Comment
	UserNo: user list index. Valid value range is [0 ~ 6].

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	User[UserNo].UserName
	string
	User name. Max length: 32.

	User[UserNo].Password
	string
	User password. Max length: 32.

	User[UserNo].PwdConfirm
	string
	User password conform. Max length: 32.

	User[UserNo].SetRight
	integer
	Set right. Bitwise value.

	User[UserNo].PreviewChannel
	integer
	Preview channel .

	User[UserNo].PlayBackChannel
	integer
	Playback channel.

	User[UserNo].BackupChannel
	integer
	Backup channel.

	User[UserNo].PlayBack
	char
	Record playback right.

	User[UserNo].Backup
	char
	Bakcup right

	User[UserNo].Preview
	char
	Preview right.

	User[UserNo].Switch
	char
	User in use.

0: not use. 1: in use.

	User[UserNo].PwdSwitch
	char
	Login password switch.

0: close.  1: open.

	User[UserNo].IsLive
	char
	User is on line.

0: off line.

1: on line.


10.3 NTP

10.3.1 GetNTPConfig
	URL Syntax
	http://<ip>/action/getSystemConfig?name=NTP

	Comment
	

	Response
	NTP.Switch=1

NTP.Server=3

NTP.Port=12

NTP.SyncPeriod=31

NTP.TimeZone=8


10.3.2 SetNTPConfig
	URL Syntax
	http://<ip>/action/setSystemConfig?name=NTP&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	NTP.Switch
	char
	Enable/Disable NTP server.

	NTP.Server
	char
	NTP server flag.

	NTP.Port
	short
	Range is [1-65535].
Port of NTP server.

	NTP.SyncPeriod
	short
	Range is [0-65535], unit is minutes

	NTP.TimeZone
	char
	Range is [0-32].
0:  "GMT+00:00"

1:  "GMT+01:00"

2:  "GMT+02:00"

3:  "GMT+03:00"

4:  "GMT+03:30"

5:  "GMT+04:00"

6:  "GMT+04:30"

7:  "GMT+05:00"

8:  "GMT+05:30"

9:  "GMT+05:45"

10:  "GMT+06:00"

11:  "GMT+06:30"

12:  "GMT+07:00"

13:  "GMT+08:00"

14:  "GMT+09:00"

15:  "GMT+09:30"

16:  "GMT+10:00"

17:  "GMT+11:00"

18:  "GMT+12:00"

19:  "GMT+13:00"

20:  "GMT-01:00"

21:  "GMT-02:00"

22:  "GMT-03:00"

23:  "GMT-03:30"

24:  "GMT-04:00"

25:  "GMT-05:00"

26:  "GMT-06:00"

27:  "GMT-07:00"

28:  "GMT-08:00"

29:  "GMT-09:00"

30:  "GMT-10:00"

31:  "GMT-11:00"

32:  "GMT-12:00"


10.4 DST
10.4.1 GetDSTConfig
	URL Syntax
	http://<ip>/action/getSystemConfig?name=DST

	Comment
	

	Response
	DST.StartHour=00:00:00
DST.EndHour=23:59:59

DST.StartDate=2015-04-12

DST.EndDate=2015-04-12

DST.StartMonth=1

DST.EndMonth=2

DST.StartWeek=0

DST.EndWeek=1

DST.StartWeekDay=1

DST.EndWeekDay=2

DST.Switch=0

DST.Mode=0

DST.HaveDstTime=1

DST.Offset=0


10.4.2 SetDSTConfig
	URL Syntax
	http://<ip>/action/setSystemConfig?name=DST&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	DST.StartHour
	string
	Dst start time. Format : hour:minute:second.

	DST.EndHour
	string
	Dst end time. Format : hour:minute:second.

	DST.StartDate
	string
	Dst start date. Format: year-month-day.

	DST.EndDate
	string
	Dst end date. Format: year-month-day.

	DST.StartMonth
	char
	Range is [0 ~ 11].


	DST.EndMonth
	char
	Range is [0 ~ 11].

	DST.StartWeek
	char
	Range is {0, 1, 2, 3, 4}.

0: the first week, 1: the second week...

	DSY.EndWeek
	char
	Range is {0, 1, 2, 3, 4}.

0: the first week, 1: the second week...

	DST.StartWeekDay
	char
	Which day start from.

 0-Sun, 1-Mon,  ... 6-Sat

	DST.EndWeekDay
	char
	Which day end of.

 0-Sun, 1-Mon,  ... 6-Sat

	DST.Switch
	char
	Enable/Disable DST (daylight saving time)

	DST.Mode
	char
	0: Week.

1: Date.

	DST.HaveDstTime
	char
	0: not DST.  1: is DST.

	DST.Offset
	
	Dst offset value.

0: 1hour.

1: 2hour.


11. Storage

11.1 CloudStorage
11.1.1 GetCloudStorageConfig
	URL Syntax
	http://<ip>/action/getSystemConfig?name=CloudStorage

	Comment
	Currently only use one cloud user. Current not support.

	Response
	Cloud.Key[0]=raysharp

Cloud.Password[0]=123456

Cloud.Key[1]=

Cloud.Password[1]=

Cloud.Key[2]=

Cloud.Password[2]=

Cloud.DirName=cloud

Cloud.FtpPath=ftp

Cloud.Switch=1

Cloud.Type=0

Cloud.TimeTrigger=0

Cloud.Version=1.2.1


11.1.2 SetCloudStorageConfig
	URL Syntax
	http://<ip>/action=setSystemConfig?name=CloudStorage&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	Currently only use one cloud user. Current not support.

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	Cloud.Key[0]
	string
	User key. Max length: 128.

	Cloud.Key[1]
	
	

	Cloud.Key[2]
	
	

	Cloud.Password[0]
	string
	User key password. Max length: 128.

	Cloud.Password1]
	
	

	Cloud.Password[2]
	
	

	Cloud.DirName
	string
	Upload direction. Max length: 32.

	Cloud.FtpPath
	string
	ftp upload path. Max length: 96.

	Cloud.Switch
	char
	Enable/Disable the cloud.

	Cloud.Type
	char
	0-DROPBOX, 1-GOOGLE

	Cloud.TimeTrigger
	char
	Trigger time

	Cloud.Version
	string
	Cloud version. Max length: 8.


11.2 Device Storage 

11.2.1 GetDevStorageConfig
	URL Syntax
	http://<ip>/action/getSystemConfig?name=DevStorage

	Comment
	

	Response
	Device.Redundancy=1
Device.OverWriteSwitch=1

Device.OverWritePeriod=6

Device.OverWriteMode=1


11.2.2 SetDevStorageConfig
	URL Syntax
	http://<ip>/action/setSystemConfig?name=DevStorage&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	

	Response
	OK or Error


	ParamName
	ParamValue type
	Description

	Device.Redundancy
	integer
	Each bit represent a disk attribute. 

0: main disk, 1: sub disk.

	Device.OverWriteSwitch
	char
	0:close,  1: open.

	Device.OverWritePeriod
	char
	Overwrite period. unit is hour.

	Device.OverWriteMode
	char
	Overwrite mode.

0: by hour,  1: by day.


12. Advance
12.1 IP Filter

12.1.1 GetIPFilterConfig
	URL Syntax
	http://<ip>/action/getSysConfig?name=IPFilter

	Comment
	FilterNo below is the banned IP list index. Valid value range is [0 ~ 15].

	Response
	IPFilter.Switch=1

IPFilter.Filtered[FilterNo ].IPAddress=192.168.1.111

IPFilter.Filtered[FilterNo ].Enable=1


12.1.2 SetIPFilterConfig
	URL Syntax
	http://<ip>/action/setConfig?name=IPFilter&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	FilterNo  is the banned IP list index. Valid value range is [0 ~ 15].

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	IPFilter.Switch
	char
	Enable/Disable access filter function.

	IPFilter.Filtered[FilterNo ].IPAddress
	string
	Filtered IP address. Max length: 32.

	IPFilter.Filtered[FilterNo ].Enable
	char
	Filtered IP address switch.

0: disable, 1: enable.


12.2 FtpUpgrade
12.2.1 GetFtpUpgradeConfig
	URL Syntax
	http://<ip>/action/getConfig?name=FtpUpgrade

	Comment
	Current not support.

	Response
	FtpUpgrade.IPAddress=192.168.1.109

FtpUpgrade.UserName=admin

FtpUpgrade.Password=123456

FtpUpgrade.FtpPath=upgrade

FtpUpgrade.ProductName=rs-ipc

FtpUpgrade.Port=21

FtpUpgrade.Switch=1

FtpUpgrade.Tips=0


12.2.2 SetFtpUpgradeConfig
	URL Syntax
	http://<ip>/action/setConfig?name=FtpUpgrade&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	Current not support.

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	FtpUpgrade.IPAddress
	string
	FTP IP Address. Max length: 32.

	FtpUpgrade.UserName
	string
	FTP user name. Max length: 32.

	FtpUpgrade.Password
	string
	FTP user password. Max length: 32.

	FtpUpgrade.FtpPath
	string
	FTP upgrade path. Max length: 64.

	FtpUpgrade.ProductName
	string
	Product name. Max length: 32.

	FtpUpgrade.Port
	short
	FTP port.

	FtpUpgrade.Switch
	char
	Enable/Disable Ftp upgrade function.

	FtpUpgrade.Tips
	char
	FTP upgrade tips.


12.3 HttpUpgrade
12.3.1 GetHttpUpgradeConfig
	URL Syntax
	http://<ip>/action/getConfig?name=HttpUpgrade

	Comment
	Current not support.

	Response
	HttpUpgrade.Port=80

HttpUpgrade.Addr=192.168.1.129

HttpUpgrade.Path=/upgrade/


12.3.2 SetHttpUpgradeConfig
	URL Syntax
	http://<ip>/action/setConfig?name=HttpUpgrade&<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	Current not support.

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	HttpUpgrade.Port
	int
	HTTP port.

	HttpUpgrade.Addr
	string
	HTTP IP Address. Max length: 64.

	HttpUpgrade.Path
	string
	HTTP path. Max length: 128.


12.4 ParamDefault
12.4.1 ColorDefault
	URL Syntax
	http://<ip>/action/colorDefault?

	Comment
	

	Response
	Color.Brightness=128
Color.Contrast=128
Color.Hue=128
Color.Saturation=128
Color.Sharpness=128


12.4.2 ImageDefault
	URL Syntax
	http://<ip>/action/imageDefault?

	Comment
	

	Response
	Camera.IRCutMode=0
Camera.IRCutDelay=2
Camera.IRCutSensitive=1
Camera.Mirror=0
Camera.Flip=0
Camera.Rotate=0
Camera.BackLightMode=1
Camera.BackLightLevel=1
Camera.R3dnrMode=1
Camera.R3dnrThreshTarget=128
Camera.DISEnable=1
Camera.DefogMode=1
Camera.DefogStrength=128
Camera.DwdrMode=1
Camera.DwdrStrength=128
Camera.GainControlMode=2
Camera.WBMode=0
Camera.Rgain=128
Camera.Ggain=128
Camera.Bgain=128
Camera.ShutterMode=0
Camera.ShutterSpeed=25
Camera.FlickerCtrl=0


12.5 System Operation
12.5.1 Reboot
	URL Syntax
	http://<ip>/action/reboot?

	Comment
	

	Response
	OK or ERROR


12.5.2 LoadDefault

	URL Syntax
	http://<ip>/action/loadDefault?<paramName>=<paramValue>[&<paramName>=<paramValue>...]

	Comment
	

	Response
	OK or ERROR


	ParamName
	ParamValue type
	Description

	Default.DisplayFlag
	char
	Display page default flag.  0 :disable; 1 : enable.

	Default.RecordFlag
	char
	Record page default flag.  0 :disable; 1: enable.

	Default.NetworkFlag
	char
	Network page default flag.  0 :disable; 1 : except network setting parameter; 2 : enable all.

	Default.AlarmFlag
	char
	Alarm page default flag. 0 :disable; 1 : enable.

	Default.DeviceFlag
	char
	Device page default flag. 0 :disable; 1 : enable.

	Default.SystemFlag
	char
	System page default flag. 0 : disable; 1 :enable.

	Default.AdvanceFlag
	char
	Advance page default flag. 0 : disable; 1 : enable.


13. Events

13.1 EventHandler
13.1.1 SubscribeEvent
	URL Syntax
	http://<ip>/action/eventManage?name=Subscribe

	Comment
	EventType value range is [1 ~ 4].

1 : motion detect

2 : Lens block

3 : Storage abnormal

4 : network abnormal

Status value range is {0, 1}.

1 : event start

0 : event stop

	Response
	EventType=1, Status=1, TriggerTime=2015-05-27 16:40:23


13.1.2 UnSubscribeEvent
	URL Syntax
	http://<ip>/action/eventManage?name=UnSubscribe

	Comment
	OK means stop receive event data.

	Response
	OK or ERROR


71

