PC TO LINUX
	Offset
	Flag
	Length
	Comment
	Example

	0
	PCEDITSIGNATURE
	6
	IPEdit packet header signature,

always ‘PCEdit’
	PCEdit’

	6
	IPFLAG
	1
	IP or netmask were modified.

1 - Changed

0 - No Changed.
	1

	7
	ETH1
	4
	The name of the interface, ‘eth1’

for typical IPCam.
	‘eth1’

	11
	IP
	4
	IP address
	192 168 11 53 in decimal,or

C0 A8 0B 35 in

hexdecimal.

	15
	MAC


	6
	MAC Address in Heximal

format.(Hexdecimal)
	00 03 02 03 45 21

	21
	NETMASK
	4
	Netmask
	255 255 0 0

	25
	GWFLAG
	1
	Gateway addresss were modified.

1 - Changed

0- No Changed.
	1

	26
	GWSIGNATURE
	2
	Gateway Address Header

Signature


	‘gw’


	28
	GATEWAY
	4
	Gateway address


	192 168 1 254 in decimal format, or C0 A8 01 FE in Hexdecimal format

	32
	CAMERANAMEFLAG
	1
	Camera name were modified

1 - Changed

0 - No Changed.


	1

	33
	CAMERANAME
	18
	Name of Camera
	‘Mycamera ‘

	51
	PORT1FLAG
	1
	HTTP Port1 was modified

1 - Changed

0 - No Changed.
	1

	56
	PORT1
	5
	HTTP Port1 number in ACSII

format.
	‘8888 ‘

	57
	XX
	1
	XX
	

	58
	XX
	5
	XX
	

	63
	DnsFLAG
	1
	Dns was modified

1 - Changed

0 - No Changed
	1

	64
	DNS1
	4
	DNS1 number in ASCII
	192 168 11 53

	68
	DNS2
	4
	DNS2 number in ASCII
	192 168 11 53

	72
	ProtocolFLAG
	1
	Protocol was modified

1 - Changed

0 - No Changed
	1

	73
	Protocol
	1
	1 - DHCP

0 - Static
	1 or 0


LINUX TO PC

	Offset
	Flag
	Length
	Comment
	Example

	0
	MCTESTSIGNATURE
	5
	mctest packet header signature,always ‘IPCam’
	‘IPCam’

	5
	IPFLAG
	1
	IP or netmask were modified.

1 - Changed

0 - No Changed.
	1

	6
	ETH1
	4
	The name of the interface, ‘eth1’for typical IPCam.
	‘eth1’

	10
	IP
	4
	IP address
	192 168 11 53 in decimal,or C0 A8 0B 35 in hexdecimal format

	14
	MAC
	6
	MAC Address in Hexdecimal

format.
	00 03 02 03 45 21

	20
	NETMASK
	4
	Netmask
	255 255 0 0

	24
	GWFLAG
	1
	Gateway addresss were modified.

1 - Changed

0- No Changed.
	1

	25
	GWSIGNATURE
	2
	Gateway Address Header Signature
	‘gw’

	27
	GATEWAY
	4
	Gateway address
	192 168 1 254 in decimal format, or C0 A8 01 FE in Hexdecimal format

	31
	CAMERANAMEFLAG
	1
	Camera name were modified

1 - Changed

0 - No Changed.
	1

	32
	CAMERANAME
	18
	Name of Camera
	Mycamera ‘

	50
	PORT1
	5
	HTTP Port1 number in ACSII format.
	8888 ‘

	55
	XX
	5
	XX
	

	60
	DNSFLAG
	1
	DNS were modified

1 - Changed

0 - No Changed.
	1

	61
	DNS1
	4
	DNS1 number in ASCII
	

	65
	DNS2
	4
	DNS2 number in ASCII
	

	69
	Protocol
	1
	1 -DHCP

0 - Static
	


