Manual v.1.1.8

HOEMDeviceSeeker_SDK

目錄

I.	Introduction	4

II.	DLL Function	5

1.	HOEM_Seeker_Init()	5

2.	HOEM_Seeker_Deinit()	5

3.	HOEM_Seeker_Search(int iTimeOut)	5

4.	HOEM_Seeker_GetCounts()	5

5.	HOEM_Seeker_GetMacString(int iID, char* cBuf)	5

6.	HOEM_Seeker_GetIPString(int iID, char* cBuf)	5

7.	HOEM_Seeker_GetIPValue(int iID)	6

8.	HOEM_Seeker_GetPort(int iID)	6

9.	HOEM_Seeker_GetGateWayString(int iID, char* cBuf)	6

10.	HOEM_Seeker_GetGateWayValue(int iID)	6

11.	HOEM_Seeker_GetNetMaskString(int iID, char* cBuf)	7

12.	HOEM_Seeker_GetNetMaskValue(int iID)	7

13.	HOEM_Seeker_GetName(int iID, char* cBuf)	7

14.	HOEM_Seeker_GetDNS1String(int iID, char* cBuf)	7

15.	HOEM_Seeker_GetDNS1Value(int iID)	8

16.	HOEM_Seeker_GetDNS2String(int iID, char* cBuf)	8

17.	HOEM_Seeker_GetDNS2Value(int iID)	8

18.	HOEM_Seeker_GetDHCP(int iID)	8

19.	HOEM_Seeker_Set(int iID, Set_Device_Info sInfo)	9

20.	HOEM_Seeker_GetType(int iID)	10

III.	ActiveX Function	11

1.	Initial()	11

2.	Deinit()	11

3.	Search(int iTimeOut)	11

4.	GetCounts()	11

5.	GetMacString(long iID)	11

6.	GetIPString(long iID)	11

7.	GetIPValue(long iID)	12

8.	GetPort(long iID)	12

9.	GetGateWayString(long iID)	12

10.	GetGateWayValue(long iID)	12

11.	GetNetMaskString(long iID)	12

12.	GetNetMaskValue(long iID)	13

13.	GetName(long iID)	13

14.	GetDNS1String(long iID)	13

15.	GetDNS1Value(long iID)	13

16.	GetDNS2String(long iID)	13

17.	GetDNS2Value(long iID)	14

18.	GetDHCP(long iID)	14

19.	SetDevice(long iID,BSTR *IPString,BSTR *NetMaskString,BSTR *GateWayString,BSTR *DNS1String, BSTR *DNS2String,BSTR *Name,long Port,bool DHCP)	14

20.	GetMacByteArray(long iID)	15

21.	GetType(long iID)	15

# Introduction

HOEMDeviceSeeker SDKis use to search Device in Intranet.

HOEMDeviceSeeker SDK include four files:

HOEMDeviceSeeker.h

HDEVICE_DEFINE.H

HDeviceSeekerX.dll

HDeviceSeekerX.lib

You can use in two method:DLL and ActiveX:

1.DLL

User should include HOEMDeviceSeeker.h and link HDeviceSeekerX.lib.

2.ActiveX

User need to register HDeviceSeekerX.dll.

ActiveX的UUID：E8828A68-A6F6-43DR-8FB8-AA05F7BB233A.

ActiveX Name:HDeviceSeekerX

# DLL Function

## HOEM_Seeker_Init()

Purpose:Initial HOEMDeviceSeeker SDK.

## HOEM_Seeker_Deinit()

Purpose:Close HOEMDeviceSeeker SDK.

## HOEM_Seeker_Search(int iTimeOut)

Purpose:Search Camera Device.

Return Value:

TRUE

FALSE

Parameter:

TimeOut		(Millisecond)

## HOEM_Seeker_GetCounts()

Purpose:Get counts after search.

Return Value:

Numbers of Device.

## HOEM_Seeker_GetMacString(int iID, char* cBuf)

Purpose:Retrive the MacAddress of a specific server in string format.

Return Value:

TRUE

FALSE

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

cBuf		The buffer pointer that use to save result.

## HOEM_Seeker_GetIPString(int iID, char* cBuf)

Purpose: Retrive the IPAddress of a specific server in string format.

Return Value:

TRUE

FALSE

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

cBuf		The buffer pointer that use to save result.

## HOEM_Seeker_GetIPValue(int iID)

Purpose: Retrive the IPAddress of a specific server in raw value format.

Return Value:

IPAddress的Raw Value.

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

## HOEM_Seeker_GetPort(int iID)

Purpose:Get HTTP port of a specific server.

Return Value:

Port

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

## HOEM_Seeker_GetGateWayString(int iID, char* cBuf)

Purpose: Retrive the Gateway of a specific server in string format.

Return Value:

TRUE

FALSE

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

cBuf		The buffer pointer that use to save result.

## HOEM_Seeker_GetGateWayValue(int iID)

Purpose: Retrive the Gateway of a specific server in raw value format.

Return Value:

GateWay的Raw Value.

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

## HOEM_Seeker_GetNetMaskString(int iID, char* cBuf)

Purpose:Retrive the netmask of a specific server in string format.

Return Value:

TRUE

FALSE

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

cBuf		The buffer pointer that use to save result.

## HOEM_Seeker_GetNetMaskValue(int iID)

Purpose: Retrive the netmask of a specific server in raw value format.

Return Value:

NetMask的Raw Value.

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

## HOEM_Seeker_GetName(int iID, char* cBuf)

Purpose:Get device name of a specific server.

Return Value:

TRUE

FALSE

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

cBuf		The buffer pointer that use to save result.

## HOEM_Seeker_GetDNS1String(int iID, char* cBuf)

Purpose: Retrive the DNS1 of a specific server in string format.

Return Value:

TRUE

FALSE

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

cBuf		The buffer pointer that use to save result.

## HOEM_Seeker_GetDNS1Value(int iID)

Purpose: Retrive the DNS1 of a specific server in raw value format.

Return Value:

DNS1的Raw Value.

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

## HOEM_Seeker_GetDNS2String(int iID, char* cBuf)

Purpose: Retrive the DNS2 of a specific server in string format.

Return Value:

TRUE

FALSE

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

cBuf		The buffer pointer that use to save result.

## HOEM_Seeker_GetDNS2Value(int iID)

Purpose: Retrive the DNS2 of a specific server in raw value format.

Return Value:

DNS2的Raw Value.

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

## HOEM_Seeker_GetDHCP(int iID)

Purpose: Get DHCP.

Return Value:

1		DHCP(Automatic)

0		Static(Manual)

-1		Access Failed

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

## HOEM_Seeker_Set(int iID, Set_Device_Info sInfo)

Purpose:Setup a specific server.

Return Value:

1		Success

0		Failed

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

sInfo	Setting information Set_Device_Info.

struct Set_Device_Info{

unsigned char cIP[4];

unsigned char cNetMask[4];

unsigned char cGateWay[4];

unsigned char cDNS1[4];

unsigned char cDNS2[4];

unsigned char cName[18];

long lPort;

bool bDHCP;

};

| cIP[4] | IP Address in Raw Value Format |
| --- | --- |
| cNetMask[4] | NetMask in Raw Value Format |
| cGateWay[4] | GateWay in Raw Value Format |
| cDNS1[4] | DNS1 in Raw Value Format |
| cDNS2[4] | DNS2 in Raw Value Format |
| cName[18] | Camera Name String |
| lPort | HTTP Port |
| bDHCP | 0:Static 1:DHCP |

## HOEM_Seeker_GetType(int iID)

Purpose: Retrive the server type of a specific server.

Return Value:

>0		Server Type

-1		Access Failed

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

# ActiveX Function

## Initial()

Purpose:Initial HOEMDeviceSeekerX.

## Deinit()

Purpose:Close HOEMDeviceSeekerX.

## Search(int iTimeOut)

Purpose: Search Camera Device.

Return Value:

TRUE

FALSE

Parameter:

TimeOut		(Millisecond)

## GetCounts()

Purpose: Get counts after search.

Return Value:

Numbers of Device.

## GetMacString(long iID)

Purpose: Retrive the MacAddress of a specific server in string format.

Return Value:

BSTR

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

## GetIPString(long iID)

Purpose: Retrive the IPAddress of a specific server in string format.

Return Value:

BSTR

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

## GetIPValue(long iID)

Purpose: Retrive the IPAddress of a specific server in raw value format.

Return Value:

long

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

## GetPort(long iID)

Purpose: Get HTTP port of a specific server.

Return Value:

long

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

## GetGateWayString(long iID)

Purpose: Retrive the Gateway of a specific server in string format.

Return Value:

BSTR

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

## GetGateWayValue(long iID)

Purpose: Retrive the Gateway of a specific server in raw value format.

Return Value:

long

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

## GetNetMaskString(long iID)

Purpose: Retrive the netmask of a specific server in string format.

Return Value:

BSTR

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

## GetNetMaskValue(long iID)

Purpose: Retrive the netmask of a specific server in raw value format.

Return Value:

long

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

## GetName(long iID)

Purpose: Get device name of a specific server.

Return Value:

BSTR

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

## GetDNS1String(long iID)

Purpose: Retrive the DNS1 of a specific server in string format.

Return Value:

BSTR

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

## GetDNS1Value(long iID)

Purpose: Retrive the DNS1 of a specific server in raw value format.

Return Value:

long

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

## GetDNS2String(long iID)

Purpose: Retrive the DNS2 of a specific server in string format.

Return Value:

BSTR

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

## GetDNS2Value(long iID)

Purpose: Retrive the DNS2 of a specific server in raw value format.

Return Value:

long

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

## GetDHCP(long iID)

Purpose: Get DHCP.

Return Value:

1		DHCP(Automatic)

0		Static(Manual)

-1		Access Failed

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

## SetDevice(long iID,BSTR *IPString,BSTR *NetMaskString,BSTR *GateWayString,BSTR *DNS1String, BSTR *DNS2String,BSTR *Name,long Port,bool DHCP)

Purpose: Setup a specific server.

Return Value:

1		Success

0		Failed

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

IPString	IP (ex. “192.168.1.123”)

NetMaskString  NetMask(ex. “255.255.255.0”)

GateWayString  GateWay(ex. “192.168.1.254”)

DNS1String  DNS1(ex. “168.95.1.1”)

DNS2String  DNS2(ex. “168.95.192.1”)

Name  Device name(ex. “IPCam”)

Port  HTTP Port(ex. 80)

DHCP  Automatic search IP(1 : Yes；0 : No)

## GetMacByteArray(long iID)

Purpose: Retrive the MacAddress of a specific server in raw value format.

Return Value:

VARIANT	An Array

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

## GetType(long iID)

Purpose: Retrive the server type of a specific server in raw value format.

Return Value:

>0		Server Type

-1		Access Failed

Parameter:

iID		Specific number of search counts(For example, if there’re 10 items found in search, the valid range is 0 ~ 9(zero-base)).

ServerType List :

| WIS & Winbond series |  |
| --- | --- |
| HLC-36p | 1x(10~19) |
| HLC-82F | 2x |
| HLC-36M | 3x |
| HWS-01HE | 4x |
| HWS-01HP | 5x |
| 8120 series |  |
| HWS-04HD | 59 |
| 830_7649 | 60 |
| HLT_860 | 61 |
| HLC-810 | 62 |
| HWS_01HD | 63 |
| Digital | 64 |
| DvrCard | 66 |
| HLV_1CI | 67 |
| 830_9655 | 68 |
| HVT_01HT | 69 |
| YudorBoard | 70 |
| 830_7720 | 71 |
| HLC_36F | 72 |
| HDR-04FE | 90 |