PC TO LINUX

| Offset | Flag | Length | Comment | Example |
| --- | --- | --- | --- | --- |
| 0 | PCEDITSIGNATURE | 6 | IPEdit packet header signature, always ‘PCEdit’ | PCEdit’ |
| 6 | IPFLAG | 1 | IP or netmask were modified. 1 - Changed 0 - No Changed. | 1 |
| 7 | ETH1 | 4 | The name of the interface, ‘eth1’ for typical IPCam. | ‘eth1’ |
| 11 | IP | 4 | IP address | 192 168 11 53 in decimal,or C0 A8 0B 35 in hexdecimal. |
| 15 | MAC | 6 | MAC Address in Heximal format.(Hexdecimal) | 00 03 02 03 45 21 |
| 21 | NETMASK | 4 | Netmask | 255 255 0 0 |
| 25 | GWFLAG | 1 | Gateway addresss were modified. 1 - Changed 0- No Changed. | 1 |
| 26 | GWSIGNATURE | 2 | Gateway Address Header Signature | ‘gw’ |
| 28 | GATEWAY | 4 | Gateway address | 192 168 1 254 in decimal format, or C0 A8 01 FE in Hexdecimal format |
| 32 | CAMERANAMEFLAG | 1 | Camera name were modified 1 - Changed 0 - No Changed. | 1 |
| 33 | CAMERANAME | 18 | Name of Camera | ‘Mycamera ‘ |
| 51 | PORT1FLAG | 1 | HTTP Port1 was modified 1 - Changed 0 - No Changed. | 1 |
| 56 | PORT1 | 5 | HTTP Port1 number in ACSII format. | ‘8888 ‘ |
| 57 | XX | 1 | XX |  |
| 58 | XX | 5 | XX |  |
| 63 | DnsFLAG | 1 | Dns was modified 1 - Changed 0 - No Changed | 1 |
| 64 | DNS1 | 4 | DNS1 number in ASCII | 192 168 11 53 |
| 68 | DNS2 | 4 | DNS2 number in ASCII | 192 168 11 53 |
| 72 | ProtocolFLAG | 1 | Protocol was modified 1 - Changed 0 - No Changed | 1 |
| 73 | Protocol | 1 | 1 - DHCP 0 - Static | 1 or 0 |

LINUX TO PC

| Offset | Flag | Length | Comment | Example |
| --- | --- | --- | --- | --- |
| 0 | MCTESTSIGNATURE | 5 | mctest packet header signature,always ‘IPCam’ | ‘IPCam’ |
| 5 | IPFLAG | 1 | IP or netmask were modified. 1 - Changed 0 - No Changed. | 1 |
| 6 | ETH1 | 4 | The name of the interface, ‘eth1’for typical IPCam. | ‘eth1’ |
| 10 | IP | 4 | IP address | 192 168 11 53 in decimal,or C0 A8 0B 35 in hexdecimal format |
| 14 | MAC | 6 | MAC Address in Hexdecimal format. | 00 03 02 03 45 21 |
| 20 | NETMASK | 4 | Netmask | 255 255 0 0 |
| 24 | GWFLAG | 1 | Gateway addresss were modified. 1 - Changed 0- No Changed. | 1 |
| 25 | GWSIGNATURE | 2 | Gateway Address Header Signature | ‘gw’ |
| 27 | GATEWAY | 4 | Gateway address | 192 168 1 254 in decimal format, or C0 A8 01 FE in Hexdecimal format |
| 31 | CAMERANAMEFLAG | 1 | Camera name were modified 1 - Changed 0 - No Changed. | 1 |
| 32 | CAMERANAME | 18 | Name of Camera | Mycamera ‘ |
| 50 | PORT1 | 5 | HTTP Port1 number in ACSII format. | 8888 ‘ |
| 55 | XX | 5 | XX |  |
| 60 | DNSFLAG | 1 | DNS were modified 1 - Changed 0 - No Changed. | 1 |
| 61 | DNS1 | 4 | DNS1 number in ASCII |  |
| 65 | DNS2 | 4 | DNS2 number in ASCII |  |
| 69 | Protocol | 1 | 1 -DHCP 0 - Static |  |