Manual v.2.3.20

HOEM_IPCam_SDK

Contents

I.	Introduction	4

1.	System Requirement	4

2.	Capability	4

3.	Record Mode	5

4.	Receive Type	5

5.	PTZ Module	5

6.	Connect Information	5

7.	Video Format	6

8.	Video Information	7

9.	Motion Information	9

10.	PTZ Information	9

11.	Video Callback Function	10

12.	Event Callback Function	10

13.	RTSP Protocol	11

14.	Camera Type	11

15.	PTZ Mouse Window Control	12

II.	Return Value	13

III.	Method	14

1.	HOEM_Cam_GetMaxChannel()	14

2.	HOEM_Cam_Init()	14

3.	HOEM_Cam_Deinit()	14

4.	HOEM_Cam_Create(HANDLE *hCam, Connect_Info cInfo, HEVENT_CALLBACK peCallback, int iContext)	14

5.	HOEM_Cam_Delete(HANDLE *hCam)	15

6.	HOEM_Cam_SetAutoReconnect (HANDLE hCam, bool bEnabled)	15

7.	HOEM_Cam_Connect(HANDLE hCam, HFRAME_CALLBACK pfCallback, int iContext, HFRAMETYPE_DLL eFrameType)	15

8.	HOEM_Cam_CleanAll()	16

9.	HOEM_Cam_SetRtspParams(HANDLE hCam, HRTSP_TYPE rType = rtTCP, char *pcUrlPath = “”, int iUdpPort = 0)	16

10.	HOEM_Cam_GetCamera_TypeChannel(HANDLE hCam, int &iType, int &iVideoChannel, int &iAudioChannel)	16

11.	HOEM_Cam_SetIPCamStreamNum(HANDLE hCam, int iVideoStream=0, int iAudioStream=0)	16

12.	HOEM_Cam_StartReceive(HANDLE hCam, HFRAMETYPE_DLL eFrameType)	17

13.	HOEM_Cam_PauseReceive(HANDLE hCam, HFRAMETYPE_DLL eFrameType)	17

14.	HOEM_Cam_ContinueReceive(HANDLE hCam, HFRAMETYPE_DLL eFrameType)	17

15.	HOEM_Cam_IsCamLive(HANDLE hCam)	18

16.	HOEM_Cam_GetCamStatus(HANDLE hCam)	18

17.	HOEM_Cam_IsSupportPTZ(HANDLE hCam)	18

18.	HOEM_Cam_GetVideo(HANDLE hCam, HVideo_Config *vConfig)	19

19.	HOEM_Cam_SetVideo(HANDLE hCam, HVideo_Config vConfig)	19

20.	HOEM_Cam_GetMotion(HANDLE hCam, HGet_Motion_Info *mInfo)	19

21.	HOEM_Cam_SetMotion(HANDLE hCam, HSet_Motion_Info mInfo)	20

22.	HOEM_CamGetPTZ(HANDLE hCam, HPTZ_Info *m_Info)	20

23.	PTZ Control	20

24.	HOEM_Cam_Execute_CGI(HANDLE hCam, char *pCmd, char *pRecvStr, unsigned long &pBufSize)	22

25.	HOEM_Cam_GetMPEG4Frame(HFrame *pFrame)	22

26.	HOEM_Cam_AskSystemFrame(HANDLE hCam)	23

27.	HOEM_Cam_DeleteFrame(HFrame *pFrame)	23

28.	HOEM_Cam_GetResolutionSet(HANDLE hCam, int iMode, int &iWidth, int &iHeight)	23

29.	HOEM_Cam_Set2WayAudio(HANDLE hCam, bool bEnable)	23

30.	HOEM_Cam_AudioIn_Init(HANDLE hCam, WAVEFORMATEX *pwfSrc)	24

31.	HOEM_Cam_SendAudioIn(HANDLE hCam, char *pAudioData, int iLenData)	25

32.	HOEM_Cam_AudioIn_Deinit(HANDLE hCam)	25

# Introduction

Include files:

HIPCam_SDK.dll

HIPCam_SDK.lib

HOEM_IPCam_SDK.h

HIPCAM_DEFINE.H

HIPCam_Error.h

IPCamLib.dll

HOEM_Cam_Init will load configuration file(IPCamconfig.txt). If the file is not exist, HIPCam_SDK will use default value. The default content includes :

Ping_Enable=n<E>

Ping_Timeout=6000<E>

Get_Timeout=7000<E>

Set_Timeout=7000<E>

UsePCTime=y<E>

ConnectLimit=6<E>

## System Requirement

      - Minimum: 32MB display card.

      - Suggest: 64MB display card or higher ( ex. NVIDIA GeForce FX 5200)

## Capability

      - Connect to IPCam Camera

      - Receive Video Frame

      - Receive Audio Frame

      - Receive FrameRate & FrameSize

      - Record

      - Detect Connection Status

      - IPCam Video Setup

      - IPCam Motion Detect Setup

      - IPCam PTZ Control

## Record Mode

Video and audio data received from HIPCam_SDK can be converted to MPEG4 Frame via HOEM_Cam_GetMPEG4Frame, then save as AVI format. Please refers to the test program(TestProject) AVISave.cpp and AVISave.h for more detail.

## Receive Type

HFRAMETYPE_DLL

| HIPCam_VIDEO | Receive Video |
| --- | --- |
| HIPCam_AUDIO | Receive Audio |
| HIPCam_EVENT | Receive Motion, Alarm or System Frame |

## PTZ Module

HPTZ_MODEL

HPTZ_NONE

HPTZ_HPT_11A

HPTZ_Panasonic_CS850

HPTZ_DynaColor_D

HLilin_PIH_7600

HPTZ_PELCO_D

HPTZ_PELCO_P

HPTZ_DMP_15H1

HPTZ_HLC_860

## Connect Information

struct Connect_Info{

char *sIP;

int iPort;

char *sUser;

char *sPass;

int iCh;

};

| sIP | IP Address |
| --- | --- |
| iPort | Connection Port |
| sUser | User Name |
| sPass | User Password |
| iCh | Channel Number(0 ~ SDK define max channel - 1) |

## Video Format

struct HFrame{

int iFrameType;

int iDataLen;

cahr* cFrameData;

FILETIME ftFrameTime;

};

| iFrameType | Frame Type | 2 | I Frame |
| --- | --- | --- | --- |
|  |  | 3 | P Frame |
|  |  | 4 | B Frame |
|  |  | 5 | Audio Frame |
|  |  | 6 | Motion Frame |
|  |  | 7 | System Frame |
|  |  | 9 | Alarm Frame |
| iDataLen | cFrameData’s length |  |  |
| cFrameData | Pointer point to Frame Data |  |  |
| ftFrameTime | Frame Time |  |  |

When iFrameType is Motion or Alarm，cFrameData refers to :

Motion Data

|  | Rect1 | Rect2 | Rect3 |
| --- | --- | --- | --- |
| 7 | Motio Start | Motion Start | Motion Start |
| 6 | Motion Stop | Motio Start | Motio Start |
| 5 | Motio Start | Motion Stop | Motio Start |
| 4 | Motion Stop | Motion Stop | Motio Start |
| 3 | Motio Start | Motio Start | Motion Stop |
| 2 | Motion Stop | Motio Start | Motion Stop |
| 1 | Motio Start | Motion Stop | Motion Stop |
| 0 | Motion Stop | Motion Stop | Motion Stop |

Alarm Data

|  | Sensor1 | Sensor2 |
| --- | --- | --- |
| 3 | Alarm Start | Alarm Start |
| 2 | Alarm Stop | Alarm Start |
| 1 | Alarm Start | Alarm Stop |
| 0 | Alarm Stop | Alarm Stop |

## Video Information

Video Information

struct HVideo_Config{

int iResolution;

int iQuality;

int iFps;

int iMode;

int iFormat;

};

| iResolution | 0 | QCIF |
| --- | --- | --- |
|  | 1 | CIF |
|  | 2 | VGA |
|  | 3 | D1 |
| iQuality | 0 | Low |
|  | 1 | Medium |
|  | 2 | Standard |
|  | 3 | High |
|  | 4 | Best |
| iFps | NTSC | 3~30 |
|  | PAL | 3~25 |
| iMode | 1 | MPEG4 |
|  | 2 | JPEG |
| iFormat | 1 | NTSC |
|  | 2 | PAL |

struct HVideo_Info{

ULONG 	uSystemType;

ULONG 	uVideoType;

int 		iWidth;

int 		iHeight;

double 	dFrameRate;

int		iAudioType;

int		iAudioChannel;

int		iAudioSampleRate;

int		iAudioBitRate;

int		iAudioFrameSize;

};

| uVideoType | 0 | JPEG |
| --- | --- | --- |
|  | 4 | MPEG4 |
| iWidth | Image Width(iWidth is negative when video is Mirror) |  |
| iHeight | Image Height(iHeight is negative when video is Flip) |  |
| dFrameRate | Frame Rate |  |
| iAudioType | 0 | HAT_PCM |
|  | 1 | HAT_ULOW_ADPCM |
|  | 2 | HAT_IMA_ADPCM |
|  | 3 | HAT_MPEG1_L2 |
|  | 4 | HAT_MPEG1_L3 |
|  | 5 | HAT_MS_ADPCM |
|  | 6 | HAT_IP_IMA_ADPCM |
| iAudioChannel | 0 | HACN_MONO |
|  | 1 | HACN_STEREO |
| iAudioSampleRate | Audio Sample Rate |  |
| iAudioBitRate | Audio Bit Rate |  |
| iAudioFrameSize | Audio Frame Size |  |

## Motion Information

Get IPCam Motion Setup

struct HGet_Motion_Info{

bool bEnabled[3];

RECT rArea[3];

int iSensitivity[3];

int iSenMinValue;

int iSenMaxValue;

};

| bEnabled[3] | Motion Detecet ON/OFF |
| --- | --- |
| rArea[3] | Motion Area |
| iSensitivity[3] | Motion Sensitive |
| iSenMinValue | Max Sensitive |
| iSenMaxValue | Min Sensitive |

Set Motion Setup

struct HSet_Motion_Info{

bool bEnabled[3];

RECT rArea[3];

int iSensitivity[3];

};

| bEnabled[3] | Motion Detecet ON/OFF |
| --- | --- |
| rArea[3] | Motion Area |
| iSensitivity[3] | Motion Sensitive |

## PTZ Information

struct HPTZ_Info{

HPTZ_MODEL iPTZType;

int iDeviceID;

int iBaudRate;

int iSpeed;

char* sPointName[10];

};

| iPTZType | HPTZ_MODEL |
| --- | --- |
| iDeviceID | Device ID |
| iBaudRate | Baud Rate |
| iSpeed | Speed |
| sPointName[10] | Preset Point Name |

## Video Callback Function

(CALLBACK *HFRAME_CALLBACK)(int iContext, unsigned char ucChannel, HFrame *pFrame, HVideo_Info VInfo)

| iContext | Sent one int to Callback( as an ID#) |
| --- | --- |
| ucChannel | Channel Number(same as the number)( if connect) |
| pFrame | Frame pointer |
| VInfo | Video Information |

## Event Callback Function

(CALLBACK * HEVENT_CALLBACK)(int iContext, HANDLE hCam, unsigned char ucChannel, int iEventNum)

| iContext | Sent one int to Callback( as an ID#) |
| --- | --- |
| hCam | Connection Handle |
| ucChannel | Channel Number(same as the number when connect) |
| iEventNum | Event |

iEventNum  :

IPCAM_OK

IPCAM_IS_FULL

IPCAM_USER_OR_PASSWS_IS_ER

IPCAM_CANT_DETECT_TYPE_ER

IPCAM_IS_OVER

IPCAM_IS_UNCONNECTED

## RTSP Protocol

HRTSP_TYPE

| HTCP | TCP Protocol |
| --- | --- |
| HUDP | UDP Protocol |

## Camera Type

| WIS & Winbond series |  |
| --- | --- |
| HLC-36p | 1x(10~19) |
| HLC-82F | 2x |
| HLC-36M | 3x |
| HWS-01HE | 4x |
| HWS-01HP | 50 |
| 8120 series |  |
| HWS-04HD new | 58 |
| HWS-04HD old | 59 |
| HLC-830_7649 | 60 |
| HLT-860_CCD | 61 |
| HLC-810_CCD | 62 |
| HWS_01HD | 63 |
| HLC-830_CCD | 64 |
| HLV-1CI_CCD | 67 |
| HLC-830_9655 | 68 |
| HVT_01HT | 69 |
| HLC-830_7720 | 71 |
| HLC-36F_CCD | 72 |
| HLC-810_9655 | 73 |
| HLC-36F_9655 | 74 |
| HLV-1CI_9655 | 75 |
| HLT-860_9655 | 76 |
| HLV-1CI_7720 | 77 |
| HLC-36F_7720 | 78 |
| HLC-810_7720 | 79 |
| HLT-860_7720 | 80 |
| HLT-870_CCD | 81 |
| FINE SpeedDome網卡 | 82 |
| HLC-7T(7M for Yudor) | 83 |
| HLC_830_10620 | 84 |
| HLC_810_10620 | 85 |
| HLV_1CI_0620 | 86 |
| HLC_36F_10620 | 87 |
| HLC_105_10620 | 88 |
| HLC_105_CCD | 89 |
| HDR-04FE | 90 |
| HDR-04DE | 91 |
| HLC_105_9655 | 121 |
| HLC_105_7720 | 122 |
| HLC_105_3X | 123 |
| HLC_840_10620 | 124 |
| HLC_840_9655 | 125 |
| HLC_840_7720 | 126 |
| 8180 series |  |
| HLC-81GR_CCD | 151 |
| NessyII |  |
| HLC-81NP | 161 |

## PTZ Mouse Window Control

struct PTZOSCInfo{

int iWidth;

int iHeight;

int iX;

int iY;

};

| iWidth | Window Width |
| --- | --- |
| iHeight | Window Height |
| iX | Mouse Coordinate X |
| iY | Mouse Coordinate Y |

# Return Value

#define IPCAM_OK								0

#define IPCAM_IS_NULL							-1

#define IPCAM_PORT_IS_ER                       -2

#define IPCAM_TYPE_IS_ER                       -3

#define IPCAM_USER_OR_PASSWD_IS_ER			-4

#define IPCAM_IS_UNCONNECTED					-12

#define IPCAM_IS_FULL							-13

#define IPCAM_NUM_LIMITED_ER					-14

#define IPCAM_CANT_DETECT_TYPE_ER			-15

#define IPCAM_IS_OVER							-100

#define IPCAM_IS_OVER_MAX_CHANNEL			-101

#define IPCAM_SOCKET_INIT_OK					1

#define IPCAM_SOCKET_WSASTARTUP_ER			-1

#define IPCAM_SOCKET_OPEN_CONFIG_ER			-2

#define IPCAM_SOCKET_CONFIG_PARAM_ER		-3

# Method

## HOEM_Cam_GetMaxChannel()

Description : To get the max channel number define by HIPCam_SDK.

Return Value : Max Channel number

## HOEM_Cam_Init()

Description : Initial HIPCam_SDK.

Return Value :

IPCAM_SOCKET_INIT_OK

IPCAM_SOCKET_WSASTARTUP_ER

IPCAM_SOCKET_OPEN_CONFIG_ER

IPCAM_SOCKET_CONFIG_PARAM_ER

## HOEM_Cam_Deinit()

Description : Close SDK.

Return Value :

IPCAM_OK

## HOEM_Cam_Create(HANDLE *hCam, Connect_Info cInfo, HEVENT_CALLBACK peCallback, int iContext)

Description : Create a connection.

Return Value :

IPCAM_OK

IPCAM_IS_NULL

IPCAM_PORT_IS_ER

IPCAM_IS_OVER_MAX_CHANNEL

Parameter :

hCam		the pointer to current connection’s HANDLE

cInfo		connect information

peCallback	Event Callback Function

iContext		the ID sent to Callback

## HOEM_Cam_Delete(HANDLE *hCam)

Description : Close a connection.

Return Value :

IPCAM_OK

IPCAM_IS_NULL

Parameter :

hCam	pointer to current connection’s HANDLE

## HOEM_Cam_SetAutoReconnect (HANDLE hCam, bool bEnabled)

Description: ON/OFF auto re-connection ( the default is on). Please set before invoke HOEM_Cam_Connec.

Return Value :

IPCAM_OK

IPCAM_IS_NULL

Parameter :

hCam		HANDLE of a disconnection

pfCallba		ON/OFF

## HOEM_Cam_Connect(HANDLE hCam, HFRAME_CALLBACK pfCallback, int iContext, HFRAMETYPE_DLL eFrameType)

Description : Connect to IPCam.(It needs 20 seconds to wait for timeout when IP is wrong.)

Return Value :

IPCAM_OK

IPCAM_IS_UNCONNECTED

IPCAM_IS_NULL

IPCAM_TYPE_IS_ER

Parameter :

hCam		HANDLE of a connection

pfCallback	Vidoe and Audio Callback Function

iContext		The ID sent to Callback

eFrameType	HFRAMETYPE_DL

## HOEM_Cam_CleanAll()

Description : Close all connections.

Return Value :

IPCAM_OK

## HOEM_Cam_SetRtspParams(HANDLE hCam, HRTSP_TYPE rType = rtTCP, char *pcUrlPath = “”, int iUdpPort = 0)

Description :  Please setup RTSP Parameter(Protocol and Steam name) before invoking HOEM_Cam_Connect.

Return Value :

IPCAM_OK

IPCAM_IS_NULL

Parameter :

hCam		HANDLE of a connection

rType		RTSP protocol

pcUrlPath	Stream Access Name

iUdpPort		(Range : 1~65535) Default : 0, system select port.

## HOEM_Cam_GetCamera_TypeChannel(HANDLE hCam, int &iType, int &iVideoChannel, int &iAudioChannel)

Description : Get the camera type, supported video and audio channel number.

Return Value :

IPCAM_OK

IPCAM_IS_NULL

Parameter :

hCam			HANDLE of a connection

iType			Camera type

iVideoChannel		Video total channels

iAudioChannel	Audio total channels

## HOEM_Cam_SetIPCamStreamNum(HANDLE hCam, int iVideoStream=0, int iAudioStream=0)

Description : Setup muti-channel number to get video and audio data.

Return Value :

IPCAM_OK

IPCAM_IS_NULL

Parameter :

hCam			HANDLE of a connection

iVideoStream		Video channel  :  0~(Total Video Channels-1)(Default 0)

iAudioStream		Audio channel  :  0~(Total Audio Channels-1)(Default 0)

## HOEM_Cam_StartReceive(HANDLE hCam, HFRAMETYPE_DLL eFrameType)

Description : Start to receive video or audio frame.

Return Value :

IPCAM_OK

IPCAM_IS_NULL

IPCAM_TYPE_IS_ER

Parameter :

hCam		HANDLE of a connection

eFrameType	HFRAMETYPE_DLL

## HOEM_Cam_PauseReceive(HANDLE hCam, HFRAMETYPE_DLL eFrameType)

Description : Pause receive.

Return Value :

IPCAM_OK

IPCAM_IS_NULL

IPCAM_TYPE_IS_ER

Parameter :

hCam		HANDLE of a connection

eFrameType	HFRAMETYPE_DLL

## HOEM_Cam_ContinueReceive(HANDLE hCam, HFRAMETYPE_DLL eFrameType)

Description : Continue receiveing

Return Value :

IPCAM_OK

IPCAM_IS_NULL

IPCAM_TYPE_IS_ER

Parameter :

hCam		HANDLE of a connection

eFrameType	HFRAMETYPE_DLL

## HOEM_Cam_IsCamLive(HANDLE hCam)

Description : Check connection .

Return Value :

TRUE

FALSE

Parameter :

hCam		HANDLE of a connection

## HOEM_Cam_GetCamStatus(HANDLE hCam)

Description : Check a connection status.

Return Value :

IPCAM_OK

IPCAM_IS_NULL

IPCAM_IS_FULL

IPCAM_USER_OR_PASSWD_IS_ER

IPCAM_CANT_DETECT_TYPE_ER

IPCAM_IS_OVER

IPCAM_IS_UNCONNECTED

Parameter :

hCam		HANDLE of a connection

## HOEM_Cam_IsSupportPTZ(HANDLE hCam)

Description : Check PTZ compatibility of current connection

Return Value :

TRUE

FALSE

Parameter :

hCam		HANDLE of a connection

## HOEM_Cam_GetVideo(HANDLE hCam, HVideo_Config *vConfig)

Description : To get IP cam’s resolution and compression quality.

Return Value :

TRUE

FALSE

Parameter :

hCam		HANDLE of a connection

vConfig		HVideo_Config’s pointer

## HOEM_Cam_SetVideo(HANDLE hCam, HVideo_Config vConfig)

Description : To setup the ipcam’s resolution and compression quality.

Return Value :

TRUE

FALSE

Parameter :

hCam		HANDLE of a connection

vConfig		HVideo_Config

## HOEM_Cam_GetMotion(HANDLE hCam, HGet_Motion_Info *mInfo)

Description : To get the IP cam’s motion configuration.

Return Value :

TRUE

FALSE

Parameter :

hCam		HANDLE of a connection

mInfo		HGet_Motion_Info’s pointer

## HOEM_Cam_SetMotion(HANDLE hCam, HSet_Motion_Info mInfo)

Description : To setup IP cam’s motion configuration.

Return Value :

TRUE

FALSE

Parameter :

hCam		HANDLE of a connection

mInfo		HSet_Motion_Info

## HOEM_CamGetPTZ(HANDLE hCam, HPTZ_Info *m_Info)

Description : To get the IP cam’s PTZ information.

Return Value :

TRUE

FALSE

Parameter :

hCam		HANDLE of a connection

m_Info		HPTZ_Info’s pointer

## PTZ Control

HOEM_CamSetPTZ_Param(HANDLE hCam, HPTZ_Info m_Info)

Description : Setup PTZ Parameter

HOEM_CamSetPTZ_Up(HANDLE hCam)

Description : Up

HOEM_CamSetPTZ_Down(HANDLE hCam)

Description : Down

HOEM_CamSetPTZ_Left(HANDLE hCam)

Description : Left

HOEM_CamSetPTZ_Right(HANDLE hCam)

Description : Right

HOEM_CamSetPTZ_UpLeft(HANDLE hCam)

Description : Left Up

HOEM_CamSetPTZ_UpRight(HANDLE hCam)

Description : Right Up

HOEM_CamSetPTZ_DownLeft(HANDLE hCam)

Description : Left Down

HOEM_CamSetPTZ_DownRight(HANDLE hCam)

Description : Right Down

HOEM_CamSetPTZ_Home(HANDLE hCam)

Description : Home

HOEM_CamSetPTZ_ZoomIn(HANDLE hCam)

Description : Zoom In。

HOEM_CamSetPTZ_ZoomOut(HANDLE hCam)

Description : Zoom Out。

HOEM_CamSetPTZ_FocusIn(HANDLE hCam)

Description : Focus In。

HOEM_CamSetPTZ_FocusOut(HANDLE hCam)

Description : Focus Out。

HOEM_CamSetPTZ_IrisOpen(HANDLE hCam)

Description : Iris Open。

HOEM_CamSetPTZ_IrisClose(HANDLE hCam)

Description : Iris Close。

HOEM_CamSetPTZ_AutoScan(HANDLE hCam)

Description : Auto Scan。

HOEM_CamSetPTZ_StopScan(HANDLE hCam)

Description : Stop Scan。

HOEM_CamSetPTZ_Type(HANDLE hCam, HPTZ_MODEL hType)

Description : Setup PTZ Module

HOEM_CamSetPTZ_Deviceid(HANDLE hCam, int iID)

Description : Setup Device ID

HOEM_CamSetPTZ_BaudRate(HANDLE hCam, int iBaud)

Description : Setup Baud Rate

HOEM_CamSetPTZ_Speed(HANDLE hCam, int iSpeed)

Description : Setup Speed

HOEM_CamSetPTZ_Point(HANDLE hCam, int iPoint)

Description : Move to preset point

HOEM_CamSetPTZ_Preset(HANDLE hCam, char* pStr)

Description : Setup preset

HOEM_CamSetPTZ_Remove(HANDLE hCam, char* pStr)

Description : Remove preset

HOEM_CamSetPTZ_UpStart(HANDLE hCam)

Description：always up until Stop。

HOEM_CamSetPTZ_DownStart(HANDLE hCam)

Description：always down until Stop。

HOEM_CamSetPTZ_LeftStart(HANDLE hCam)

Description：always left until Stop。

HOEM_CamSetPTZ_RightStart(HANDLE hCam)

Description：always right until Stop。

HOEM_CamSetPTZ_Stop(HANDLE hCam)

Description：PTZ Stop Command。

HOEM_Cam_SetPTZ_Move(HANDLE hCam, PTZOSCInfo Info)

Description : Setup PTZ Mouse Window Control

Return Value :

TRUE

FALSE

Parameter :

hCam		HANDLE of a connection

m_Info		HPTZ_Info

hType		HPTZ_MODEL

iID			Device ID

iBaud		Baud Rate

iSpeed		Speed

iPoint		Preset point

pStr			Preset Point Name(example  :  “1 : Door”)

PTZOSCInfo	PTZ Mouse Window Control

## HOEM_Cam_Execute_CGI(HANDLE hCam, char *pCmd, char *pRecvStr, unsigned long &pBufSize)

Description : Use CGI to setup IPCam.

Return Value :

TRUE

FALSE

Parameter :

hCam		HANDLE of a connection

pCmd		CGI command

pRecvStr		return data

pBufSize		return data length(Set pBufSize as pRecvStr’s length before use this

function)

## HOEM_Cam_GetMPEG4Frame(HFrame *pFrame)

Description : Convert pFrame’s format to MPEG4 format.

Return Value :

TRUE

FALSE

Parameter :

pFrame		HFrame’s pointer. When function call completed, pframe will be the

MPEG4 Frame

## HOEM_Cam_AskSystemFrame(HANDLE hCam)

Description : Retrieve system frame of current time.

Return Value :

HFrame’s Pointer	User should clean this frame buffer by

HOEM_Cam_DeleteFrame after usage

Ex.	HFrame *pSysFrame;

delete pSysFrame->cFrameData;

delete pSysFrame;

NULL				Can’t get System Frame

Parameter :

hCam		HANDLE of a connection

## HOEM_Cam_DeleteFrame(HFrame *pFrame)

Description : Delete frame buffer received from HOEM_Cam_AskSystemFrame

Return Value :

True		delete success

False	delete failed

Parameter :

pFrame	HFrame’s Pointer

## HOEM_Cam_GetResolutionSet(HANDLE hCam, int iMode, int &iWidth, int &iHeight)

Description : To get resolution of iMode’s image width & image height.

Return Value :

True		access success

False	access failed

Parameter :

hCam		HANDLE of a connection

iMode		resolution mode

iWidht		iMode’s image width

iHeight		iMode’s image height

## HOEM_Cam_Set2WayAudio(HANDLE hCam, bool bEnable)

Description : Setup two way audio function, supported multiple IP Cams.

Return Value :

IPCAM_2WAYAUDIO_OK

IPCAM_2WAYAUDIO_LINE_BUSY

IPCAM_2WAYAUDIO_LINE_CONNECT_ER

IPCAM_2WAYAUDIO_LINE_REQUEST_ER

IPCAM_2WAYAUDIO_SEND_HEADER_ER

IPCAM_2WAYAUDIO_SEND_DATA_ER

IPCAM_2WAYAUDIO_SOCKET_ER

Parameter :

hCam		HANDLE of a connection

bEnable		true : enable

false : disable

## HOEM_Cam_AudioIn_Init(HANDLE hCam, WAVEFORMATEX *pwfSrc)

Description : Initial Send Audio Wave In

Return Value :

IPCAM_IS_NULL

IPCAM_2WAYAUDIO_OK

IPCAM_2WAYAUDIO_LINE_BUSY

IPCAM_2WAYAUDIO_LINE_CONNECT_ER

IPCAM_2WAYAUDIO_LINE_REQUEST_ER

IPCAM_2WAYAUDIO_SEND_HEADER_ER

IPCAM_2WAYAUDIO_SEND_DATA_ER

IPCAM_2WAYAUDIO_SOCKET_ER

Parameter :

hCam	HANDLE of this connection

pwfSrc	WAVEFORMATEX's pointer(Only can receive audio format like below)

WAVEFORMATEX awSrc; memset(&awSrc, 0, sizeof(awSrc)); awSrc.wFormatTag = WAVE_FORMAT_PCM; awSrc.nChannels = 2; awSrc.wBitsPerSample = 16; awSrc.nSamplesPerSec = 16000; awSrc.nBlockAlign = awSrc.wBitsPerSample * awSrc.nChannels / 8; awSrc.nAvgBytesPerSec = awSrc.nSamplesPerSec * awSrc.nBlockAlign; awSrc.cbSize = 0;

## HOEM_Cam_SendAudioIn(HANDLE hCam, char *pAudioData, int iLenData)

Description : Send Audio Wave In

Return Value :

IPCAM_IS_NULL

IPCAM_2WAYAUDIO_OK

IPCAM_2WAYAUDIO_SEND_HEADER_ER

IPCAM_2WAYAUDIO_SEND_DATA_ER

Parameter :

hCam		HANDLE of this connection

pAudioData	WAVEHDR's lpData(Please set lpData size amount to 2041*4)

iDataLen		length of pAudioData

## HOEM_Cam_AudioIn_Deinit(HANDLE hCam)

Description : End Send Audio Wave In function

Return Value :

IPCAM_IS_NULL

IPCAM_2WAYAUDIO_OK

Parameter :

hCam		HANDLE of this connection

## HOEM_Cam_GetH264Frame(HFrame *pFrame)

Description : Convert pFrame’s format to H264 format.

Return Value :

TRUE

FALSE

Parameter :

pFrame		HFrame’s pointer. When function call completed, pframe will be the

H264 Frame