IPCamX Manual v2.3.20

Catalogue

I.	IPCamX CLSID	3

II.	Property	3

III.	Method	6

IV.	Event	11

Abstract

IPCamX is an ActiveX component to connect to IPCam Camear. IPCamX which has two major capability 1.In Time Live Watch and 2.PlayBack Video Record , Program Designer can use IPCamX and IPCam Camera to build a Digital Video Recorder System.
(NOTE:When use VB to write program, if you use event function to call the other function in IPCamX, it will cause the error occurred.)

# IPCamX CLSID

CLSID : (de258a6b-14b5-49de-9a69-862c0ff7bb03)

# Property

|  | Description |
| --- | --- |
| IPAddress : String | IPCam Camera Address EX. "192.168.1.1" "www.hinet.net" |
| Port : int | Connection Port EX. 80 |
| Account : String | User Login Name EX. " Admin" |
| Password : String | User Login Password EX. " Admin" |
| CameraTitle : String | The Camera Name Showed on IPCamX picture EX. " Front Door" |
| ShowTitle : short | Show the Camera Name on IPCamX picture |
| ShowTime : short | Show the Video Time on IPCamX picture |
| SetIOnly : short | Show I Frame only 1 : Show I Frame without showing B, P Frame, can reduce the CPU time, but the Image will be discontinous. 0 : All Frame will be decompressed and play. |
| EnablePopupMenu : short | Determine User can rightclick the mouse to setup Camera. 1 : User can setup Camera Connection. 0 : User can't setup Camera Connection. |
| FrameBufSize : int | Quality of Frame that IPCamX can be stored, when FrameSize of IPCamX is greater than FrameBufSize, the continue frame will be given up, until FrameSize of IPCamX is smaller than FrameBufSize. |
| PlayAudio : short | Determine this IPCamX can play Audio. 1 : Turn on Audio 0 : Turn off Audio |
| ToolMode : int | Determine this IPCamX is for Live ,PlayBack or Decode Video. 0 : PlayBack Record Video 1 : Live Showing 2 : Decode Frame to Bitmap |
| UndecodeFrameSize : int | Use UndecodeFrameSize to get the Frame Size that have not decode by IPCamX. |
| RtspType : int | Rtsp Protocol 0 : TCP 1 : UDP |
| RtspUrlPath : String | Stream Access Name Ex. “v1” |
| VideoChannel : int | Setup muti-channel number to get video data (0 ~ Total Video Channel - 1) |
| AudioChannel : int | Setup muti-channel number to get audio data (0 ~ Total Audio Channel - 1) |
| UseMenu : short | Determine use IPCamX’s PopupMenu. 1 : use 0 : not use |
| VideoFlip : short | Setup video flip 1 : video flip 0 : video not flip |
| VideoMirror : short | Setup video mirror 1 : video mirror 0 : video not mirror |
| EnableMousePTZ: short | Enable Mouse Control PTZ 1 : Enable 0 : Disable |
| EnableDBFull: short | Enabel Mouse Double Click to FullScreen 1 : Enable 0 : Disable |
| OSDString : String | OSD String |
| OSDPosX | Coordinate X |
| OSDPosY | Coordinate Y |
| OSDHeight | Character Height |
| OSDBold | Character Bold |
| OSDItalic | Character Italic |
| OSDColorR | Red Color(0~255) |
| OSDColorG | Green Color (0~255) |
| OSDColorB | Blue Color (0~255) |
| ConnectFullText | Use to instead "Connect Full" string. |
| LimitFullText | Use to instead "Limitation Full" string. |
| CantDetectText | Use to instead "Can't Detect Type" string. |
| ConnectFailedText | Use to instead "Connect Failed" string. |
| AuthorizeFailedText | Use to instead "Authorize Failed" string. |
| DisconnectText | Use to instead "Disconnect" string. |
| TimeTextX | Time display X coordinate. |
| TimeTextY | Time display Y coordinate. |
| TimeTextFontHeight | Time display Height. |
| TitleTextX | Title display X coordinate. |
| TitleTextY | Title display Y coordinate. |
| TitleTextFontHeight | Title display Height. |
| FullTimeTextX | Time display X coordinate in full screen. |
| FullTimeTextY | Time display Y coordinate in full screen. |
| FullTimeTextFontHeight | Time display Height in full screen. |
| FullTitleTextX | Title display X coordinate in full screen. |
| FullTitleTextY | Title display Y coordinate in full screen. |
| FullTitleTextFontHeight | Title display Height in full screen. |

# Method

InitCamera()

Purpose: Initial IPCamX Description: Each IPCamX should be Initial before use.

DeinitCamera()

Purpose: Close IPCamX Description: Please Close each IPCamX efore shutdown the program.

SetAutoReconnect(short bEnabled)

Purpose: Enable/Disable auto reconnection ( default is on). Please setup before invoking ConnectCamera. Parameter：bEnabled : true(enable) false(disable)

ConnectCamera()

Purpose: Connect to Camera

DisconnectCamera()

Purpose: Disconnect Camera

StartReceiveData()

Purpose: Start Receive Data Description: Use StartReceiveData() to Start Receive Data after Connect to Camera.

SetRtspParam()

Purpose：Setup Rtsp Parameter Description：Use Rtsp method before invoking StartReceive

PauseReceiveData()

Purpose: Pause Receive Camera Data

ContinueReceiveData()

Purpose: Continue Receive Camera Data

StartPlay()

Purpose: Start Play

StopPlay()

Purpose: Stop Play and Clear All Frames in IPCamX

PausePlay()

Purpose: Pause Play

StepPlay()

Purpose: Step Play, only suitable for PlayBack.

PushPlayBackFrameByBSTR(long FrameType, long DataLength, BSTR* DataBSTR, long HighDateTime, long LowDateTime, long SpareParts)

Purpose: In the PlayBack Mode, please push SystemFrame(7) to the IPCamX before starting a new PlayBack. In the Decode Mode, user can push frame data to IPCamX, IPCamX will decode. Bitmap Data and pass result by event OnDecodeBitmapByBSTR.

PushPlayBackFrameByPointer(long FrameType, long DataLength, long *DataPointer, long HighDateTime, long LowDateTime)

Purpose: In the PlayBack Mode, Please push SystemFrame(7) to the IPCamX before starting a new PlayBack. Decode Mode, user can push frame data to IPCamX, IPCamX will decode Bitmap Data and pass result by event OnDecodeBitmapByPointer.

SetVideoSetup(long Resolution, long Quality)

Purpose: Setup Camera Resolution and Quality. Parameter：Resolution:0(QCIF) 1(CIF) 2(VGA) 3(D1) Quality: 0(Low) 1(Medium) 2(Standard) 3(High) 4(Best)

SetPtzUP() SetPtzDOWN() SetPtzLEFT() SetPtzRIGHT() SetPtzZOOMIN() SetPtzZOOMOUT() SetPtzFOCUSIN() SetPtzFOCUSOUT() SetPtzHOME() SetPtzUR() SetPtzUL() SetPtzDR() SetPtzDL() SetPtzIRISOPEN() SetPtzIRISCLOSE() SetPtzAUTOSCAN() SetPtzSTOPSCAN() SetPtzTYPE(long PTZType) SetPtzDEVICEID(long DEVICEID) SetPtzBAUDRATE(long BAUDRATE) SetPtzSPEED(long SPEED) SetPtzPOINT(long POINT) SetPtzPRESET(long POINT, LPCTSTR Name) SetPtzREMOVE(long POINT, LPCTSTR Name)

Purpose : PTZ Control Method

SetVideoWindow(long iLeft, long iTop, long iWidth, long iHeight)

Purpose : Use SetVideoWindow to Move IPCamX or Change the Size of IPCamX.

PushToGetDecodeDataByPointer(long DataLength, long * DataPointer)

Purpose : The image data received from OnFrameComingByPointer can be sent to IPCamX to get MPEG4 or Motion-JPEG or H264 image. This method is suitable for BCB or VC.

PushToGetDecodeDataByBSTR(long DataLength, BSTR* DataBSTR, long SpareParts)

Purpose : The image data received from OnFrameComingByPointer can be sent to IPCamX to get MPEG4 or Motion-JPEG or H264 image, also, the decoded data is received from event of OnGetDecodeDataByBSTR. This method is suitable for VB.

GetDecodeBitampByPointer()

Purpose : In Decode Mode, the image data received from OnFrameComingByPointer can be sent to IPCamX by PushPlayBackFrameByPointer. The Bitmap data will pass by event OnDecodeBitmapByPointer. This method is suitable for BCB and VC.

GetDecodeBitmapByBSTR()

Purpose : In Decode Mode, the image data received from OnFrameComingByBSTR can be sent to IPCamX by PushPlayBackFrameByBSTR. The Bitmap data will pass by event of OnDecodeBitmapByBSTR. This method is suitable for VB.

LiveSnapShot()

Purpose : In Live Mode, You can get Live Image decode Bitmap. The Bitmap data will pass by event OnSnapShotByPointer and OnSnapShotByBSTR.

Set2WayAudioON()

Purpose : Turn on two-way audio. When there are several IPCamXs, only one channel’s two-way audio can be opened.

Set2WayAudioOFF()

Purpose : Turn off two-way audio, can support multiple IP Cams.

ExecuteCGIByBSTR(BSTR CmdStr, long CmdLen)

Purpose : When runs CGI function, the result will be returned from OnExecuteCGIBSTR. This method is suitable for VB. Parameter : CmdStr : CGI string CmdLen : The length of CmdStr

ExecuteCGIByPointer(LPCTSTR CmdStr, long CmdLen, long *ReturnStr, long *ReturnLen)

Purpose : When runs CGI functin, the result will be saved in ReturnStr. This method is suitable for BCB and VC. Parameter: CmdStr : CGI String CmdLen : The length of CmdStr. ReturnStr : Returned result. ( Please set proper size for keeping returned string) ReturnLen : The length of the returned result.

SnapShotToFile(BSTR FileName)

Purpose : Capture Screen and Save to JPG. Parameter: FileName : Path (ex. C:\\Test.jpg)

GetCGI(LPCTSTR CGIName, LPCTSTR Param)

Purpose : Execute CGI and get return value of Param. Parameter: CGIName : CGI Name Param : Specific Parameter

SetCGI(LPCTSTR CGIName)

Purpose : Execute CGI. Parameter: CGIName : CGI Command

SaveAVI(short UseSaveDialog, LPCTSTR FileName)

Purpose :Save AVI. Parameter: UseSaveDialog : 1(Open SaveDialog), 0(Use FileName) FileName : File Name

StopAVI()

Purpose :Stop AVI.

OSDShow

Purpose : Show OSD.

OSDClose

Purpose : Close OSD.

EnableEncrypt()

Purpose : Enable Channel Encryption Key

DisableEncrypt

Purpose : Disable Channel Encryption Key

# Event

OnFrameComingByBSTR(long FrameType, long DataLength, BSTR* DataBSTR, long HighDateTime, long LowDateTime, long SpareParts)

Description : Each Frame receive from Camera will pass to user by BSTR format. User should storage each frame in order if need to PlayBack Video. Parameter : FrameType : I(2) P(3) B(4) Audio(5) SYSTEM(7) DataLength: FrameData's Length DataBSTR: FrameData's Data HighDateTime & LowDateTime: dwHighDateTime of FILETIME and dwLowDateTime of FILETIME SpareParts: Frame will be split when it is too large, SpareParts mean how many spare parts after this part.

OnFrameComingByPointer(long FrameType, long DataLen, long *DataPointer, long HighDateTime, long LowDateTime)

Description: Each Frame receive from Camera will pass to user by Pointer. User should storage each frame in order if need to PlayBack Video. Parameter : FrameType : I(2) P(3) B(4) Audio(5) SYSTEM(7) DataLength: FrameData's Length DataPointer: FrameData's Data HighDateTime & LowDateTime: dwHighDateTime of FILETIME and dwLowDateTime of FILETIME

OnMotion(bool MotionFlag)

Description : When Live Video is detected Motion, IPCamX will pass this Event to User. Parameter: MotionFlag : true is Motion Start false is Motion Stop

OnAlarm(bool AlarmFlag)

Description：When Live Video has Alarm occur, IPCamX will pass this Event to User. Parameter：AlarmFlag : true is Alarm Start false is Alarm Stop

OnWarningEvent(long InfoMsg)

Description：When IPCamX has error or warning, IPCamX will pass this Event to User. Parameter：InfoMsg : 1(Msg_BufFull) : FrameSize of IPCamX is greater than FrameBufSize. 2(Msg_Connect) : Connect Success. 3(Msg_Connection_Full) : IPCam Connection is Full. 4(Msg_Limitation_Full) : Limitation is Full. 5(Msg_Cant_Detect_Type) : Can Not Detect Camera Type. 6(Msg_Unconnected) : IPCam Connection is Failed. 7(Msg_Authorize_Error) : User or Password is Error. 8(Msg_Disconnect) : Disconnect IPCam. 9(Msg_2WayAudio_Line_Busy) 10(Msg_2WayAudio_Line_Connect_Er) 11(Msg_2WayAudio_Line_Request_Er)

OnGetCamVideo(long iResolution, long iQuality)

Description：When IPCamX has finish get video setup from IPCam, IPCamX will pass Resolution and Quality to User. Parameter：iResolution : IPCam Resolution Setup. iQuality : IPCam Quality Setup.

OnGetDecodeDataByBSTR(long DataLength, BSTR* DataBSTR,long SpareParts)

Description : This event will pass the standard MPEG4 or Motion-JPEG or H264 Frame Data to User. Parameter : DataLength: FrameData's Length DataBSTR: MPEG4 or Motion-JPEG Frame Data SpareParts: Frame will be split when it is too large, SpareParts mean how many spare parts after this part.

OnDecodeBitmapByBSTR(long StrLength, BSTR* BitmapData,long SpareParts)

Purpose : In decode Mode, this event will pass the Bitmap Data to User. Parameter : StrLength: Bitmap’s Length BitmapData: Bitmap's Data SpareParts: Frame will be split when it is too large, SpareParts mean how many spare parts after this part.

OnDecodeBitmapByPointer(long DataLength, long * BitmapData)

Purpose : In decode Mode, this event will pass the Bitmap Data to User. This is suitable for BCB and VC. Parameter : DataLength: Bitmap’s Length. BitmapData: Bitmap’s Data.

OnSnapShotByPointer(long DataLength, long * BitmapData)

Purpose : In Live Mode, this event will pass the Bitmap Data to User. This is suitable for BCB and VC. Parameter : DataLength: Bitmap’s Length. BitmapData: Bitmap’s Data.

OnSnapShotByBSTR(long StrLength, BSTR* BitmapData,long SpareParts)

Purpose : In Live Mode, this event will pass the Bitmap Data to User. Parameter : StrLength: Bitmap’s Length BitmapData: Bitmap's Data SpareParts: Frame will be split when it is too large, SpareParts mean how many spare parts after this part.

OnExecuteCGIByBSTR(long ReturnLen, BSTR* ReturnStr)

Purpose : When runs “ExecuteCGIByBSTR” , the result will be returned by this event. Parameter : ReturnLen : The length of returned result. ReturnStr : The returned result of the CGI.

OnGetTypeChannel(long Type, long VChannels, long AChannels)

Purpose : When IPCamX has finish get video setup from IPCam, IPCamX will pass camera type, supported video and audio channel number to User. Parameter : Type : Camera type. VChannels : Video total channels. AChannels : Audio total channels.

OnMouseLDown

Purpose : Mouse left button down.

OnMouseLUp

Purpose : Mouse left button up.

OnMouseLDblClk

Purpose : Mouse left button double click.

OnMouseRDown

Purpose : Mouse right button down.

OnMouseRUp

Purpose : Mouse right button up.