Manufacture:

iCATCH

Model name:
RAV-1613A-N

| Model |  | RAS-1613A-N |
| --- | --- | --- |
| Product Name |  | RAS-1613A-N |
| Video | Input | 720p@30fps, 720p@25fps Analog HD 960H / D1 (Service in pairs)* |
|  | Output | 1 HDMI with Audio and 1 VGA ( Max. 1920x1080) 1 Main BNC for D1/960H |
| Audio | Input | 16 RCA |
|  | Output | 1 RCA |
| System | Operating System | Embedded Linux OS |
|  | SATA Devices | Max. 4 x SATA Devices (HDDx3 + eSATAx1; HDDx2 + Optical Device x1 + eSATAx1) |
|  | External HDD | YES ( eSATA ) |
|  | Sensor In (NO, NC) Relay out | Sensor-in x 16 Relay-out x 2 |
|  | Serial Interface | RS-485 |
|  | USB | USB2.0 x 2 |
|  | Dimension ( mm ) | 430(w)x55(h)x293(d) |
|  | Weight ( w/o HDD/DVD±RW ) | 1.85 KG |
|  | Power Supply | 12V 5.83A ,70W |
|  | Power Rating | Adapter 110/220V |
| Display | Split screen | 1, 4, 9, 13, 16, SEQ. |
|  | Auto Channel Switching | Switching time : 1 ~ 99 sec |
|  | Mode | Zoom ( Digital x2~ x5), PIP, Auto SEQ. |
| Recording | Recording Throughput | 16 x 720P@15fps(1280x720) 16 x 960H@30fps |
|  | Mode | Continuous, Event, Schedule |
| Playback | Search mode | Date/time, Event |
|  | Snapshot | YES |
| Network | Network Interface | 10/100/1000 Ethernet |
|  | Protocol | TCP/IP, SMTP, HTTP, DHCP, PPPoE(ADSL), NTP, DDNS |
|  | Remote Software | iWatch DVR2 App, IE Browser, iCMS |
|  | Account | 1 Admin + 10 Users |
|  | Mobile | Windows Phone 7.8, iPhone, iPad, Android |
| Backup | Interface | USB 2.0 |
|  | Audio Backup | YES |
|  | Backup Type | Network, Ext. USB flash device , SATA optical DVD..etc |
| Alarm | Event Log | Up to the 10000 events (Sensor, Motion, video Loss, power on, remote login, logout, HDD error, HDD full) |
| OSD & DVR Control | GUI | 32-bit True Color |
|  | Multi-Language | Supported |
|  | USB Mouse | YES |
|  | Touch Panel | Touch Panel LCD Monitor (USB Interface, EETI Chipset Only) |
|  | System Upgrade | USB Flash Drive, iWatch DVR |